Section 01 4731 
Flange Assembly for Pressure Systems

  - *************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 3 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Pressure Safety POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

      - ==========================================================

      - This is not a typical LANL master.  It is verbiage and examples of flanged-joint design/fab final design that the Design Agency must convey to the Constructor/fabricator.  Conveying this can be accomplished in several ways, namely:

        - Option 1:  Include this section in the specification package while ensuring that (a) ​<< 01 4731 - Flange Assembly for Pressure Systems>>​ is invoked by the technical sections (in Div. 22, 23, 40, etc.) and (b) ensure those Sections are clear that the submittals discussed here are always sent in associated to them (i.e., per the affected Div. 02-48 section).

        - Option 2:  Do not include this section but rather include its applicable material on the drawings and/or in the Div. 02-48 specification sections (again, delete all references to this Section in Specification package) (NOTE: Option 2 is the least desirable option).

      - This document captures LANL’s ASME B31 flange assembly requirements once (rather than in multiple sections) for LANL’s convenience in maintaining the requirements.  It is provided as an aid to the Design Agency for providing a complete, code-compliant design that LANL can review and accept during the design phase.  Delegation and deferment of said design requires LANL Project Engineer and Pressure Safety SME prior approval.

      - This section potentially affects (but is not limited to) the following related LANL Masters:

        - Section 11 5311.18 - Glovebox Atmosphere Regenerable Purification Systems

        - Section ​<< 22 1500 - Compressed-Air Systems>>​

        - Section ​<< 23 1123 - Facility Natural-Gas Piping>>​

        - Section ​<< 23 2113 - Hydronic Piping>>​

        - Section ​<< 23 2215 - Steam and Condensate Heating Piping and Specialties>>​

        - Section ​<< 23 2300 - Refrigerant Piping>>​

        - Section ​<< 25 5000 - Integrated Automated Facility Controls>>​

        - Section ​<< 33 5100 - Natural Gas Distribution>>​

        - Section ​<< 33 6300 - Steam Energy Distribution>>​

        - Section ​<< 40 0504 - Process Piping>>​

      - This template was developed to meet the requirements for ASME B31.3-2022, Fluid Service Category D and Normal, except for strong oxidizers like oxygen, high temperature service, severely cyclic conditions, and cryogenic service; and ASME B16.5 flanges.  Application of this template to a different fluid service or edition of ASME B31.3 and a different flange specification (other than ASME B16.5) will require a detailed review of the code and LANL ESM Chapter 17 - Pressure Safety by the designer.  See ASME B31.3, Appendix M, Fig. M300 for classifying fluid services for fluids not listed.

      - The designer is encouraged to read and apply ASME B16.5 and ASME PCC-1.

      - The designer is encouraged to review the DOE Handbook DOE-HDBK-1132, Design Considerations, and the LANL ESM Chapter 17 - Pressure Safety.

  - *************************************************************************************************************

Part 1 - GENERAL

Section Includes

General requirements for flange assembly.

        - ******************************************************************************

        - The following subparagraphs are designer choices based on project-specific conditions.

        - ******************************************************************************

Flange Joint

Externally Threaded Fasteners

Internally Threaded Fasteners

Washers

Metallic Gaskets

Non-Metallic Gaskets

Electrical Isolation Kit

Fastener Assembly Lubricant

Gasket Spray Adhesive

      - ***************************************************************************************

      - Designer shall include the specification Section(s) (fourth paragraph to the last paragraph in Article) below that include flange joints covered by this Section.

      - Div 11 spec sections are not in SpecLink and need to be downloaded from the Engineering Standards specification website to include in the specification package, as applicable.

      - ***************************************************************************************

Related Requirements

Section << 01 2500 - Substitution Procedures>>​​

Section << 01 3300 - Submittal Procedures>>

Section << 01 4000 - Quality Requirements - Non-Nuclear>>​.

Section 11 5311.18 - Glovebox Atmosphere Regenerable Purification Systems

Section << 22 0813 - Testing Piping Systems>>

Section << 22 1500 - Compressed-Air Systems>>

Section << 23 1123 - Facility Natural-Gas Piping>>

Section << 23 2113 - Hydronic Piping>>

Section << 23 2215 - Steam and Condensate Heating Piping and Specialties>>

Section << 23 2300 - Refrigerant Piping>>​

Section << 25 5000 - Integrated Automated Facility Controls>>

Section << 33 5100 - Natural Gas Distribution>>​

Section << 33 6300 - Steam Energy Distribution>>

Section << 40 0504 - Process Piping>>

    - **********************************************************************************************

      - In the listing below, designer must eliminate standards that are not applicable to the project and add standards that are applicable.

      - The following reference standards are not cited in this Section outside of Article Reference Standards and/or notes to the designer. It is a designer choice to include them if they are applicable to the project:

        - <<ASME B16.1>>

        - <<ASME B16.42>>

        - <<ASME B16.47>>

        - <<ASTM A395/A395M>>

        - <<ASME BPVC-VIII-1>>

        - <<ASME PCC-1>>

    - **********************************************************************************************

Reference STANDARDS

      - ​​<< All national standards invoked herein shall be taken to be the edition in effect for the code of record listed in the drawing General sheet(s), Project Pressure Safety Implementation Plan or equivalent document, unless noted otherwise.; or  All national standards invoked below and herein shall be taken to be the latest edition.>>​​

ASME B1.1 - Unified Inch Screw Threads (UN, UNR, and UNJ Thread Forms).

ASME B16.1 - Gray Iron Pipe Flanges and Flanged Fittings: Classes 25, 125, and 250.

ASME B16.5 - Pipe Flanges and Flanged Fittings:  NPS 1/2 Through NPS 24 Metric/Inch Standard.

ASME B16.20 - Metallic Gaskets for Pipe Flanges.

ASME B16.21 - Nonmetallic Flat Gaskets for Pipe Flanges.

ASME B16.42 - Ductile Iron Pipe Flanges and Flanged Fittings: Classes 150 and 300.

ASME B16.47 - Large Diameter Steel Flanges: NPS 26 through NPS 60, Metric/Inch Standard.

ASME B18.2.1 - Square, Hex, Heavy Hex, and Askew Head Bolts and Hex, Heavy Hex, Hex Flange, Lobed Head, and Lag Screws (Inch Series).

ASME B18.2.2 - Nuts for General Applications: Machine Screw Nuts; and Hex, Square, Hex Flange, and Coupling Nuts (Inch Series).

ASME B31.1 - Power Piping.

ASME B31.3 - Process Piping.

ASME B31.5 - Refrigeration Piping and Heat Transfer Components.

ASME B31.8 - Gas Transmission and Distribution Piping Systems.

ASME B31.9 - Building Services Piping.

ASME BPVC-VIII-1 - Boiler and Pressure Vessel Code, Section VIII, Division 1: Rules for Construction of Pressure Vessels.

ASME PCC-1 - Pressure Boundary Bolted Flange Joint Assembly.

ASTM A182/A182M - Standard Specification for Forged or Rolled Alloy and Stainless Steel Pipe Flanges, Forged Fittings, and Valves and Parts for High-Temperature Service.

ASTM A193/A193M - Standard Specification for Alloy-Steel and Stainless Steel Bolting for High Temperature or High Pressure Service and Other Special Purpose Applications.

ASTM A194/A194M - Standard Specification for Carbon Steel, Alloy Steel, and Stainless Steel Nuts for Bolts for High Pressure or High Temperature Service, or Both.

ASTM A395/A395M - Standard Specification for Ferritic Ductile Iron Pressure-Retaining Castings for Use at Elevated Temperatures.

ASTM F436/F436M - Standard Specification for Hardened Steel Washers Inch and Metric Dimensions.

NFPA 54 - National Fuel Gas Code.

ACTION Submittals

      - NOTE: The following information shall be submitted for all relevant components to the Div 02-48 section to which they apply. Submittals shall be in accordance with Section << 01 3300 - Submittal Procedures>>.

      - Before fabrication, submit flange assembly procedures.

Before fabrication, submit calibration records (e.g., torque wrenches, profilometer, calipers etc.) that will be used for flange assembly in accordance with Section << 01 4000 - Quality Requirements - Non-Nuclear>>.

Prior to receiving materials, submit the Material Control Procedure described in Article Quality Control and Quality Assurance, if required.

Quality CONTROL AND QUALITY Assurance

Material Control Procedure (in accordance with management level requirements)

Work shall be performed in accordance with a LANL-approved Material Control Procedure.  This procedure shall describe the control methods and documentation used to handle and monitor the use of controlled materials (piping component, fasteners, and welding filler rod and other components).

The procedure shall follow manufacturer’s requirements for storage and handling.

The procedure shall also address procurement controls, segregation of materials, and traceability of materials from receipt at the shop through processing and final assembly.

Examination, Inspection, and Testing

For the purposes of this section, the Subcontractor is responsible for all tasks identified as examination and testing.  Certified welding inspector (CWI) activities are considered examination.  Owner’s Inspector activities are considered inspection.

The Subcontractor shall perform examination and testing to verify the conformance to the specified requirements defined in PART 2 of this Section, as well as any Subcontractor requirements as defined as part of the Subcontractor’s QA/QC program.

The Subcontractor shall provide a “Test and Inspection Plan” indicating all testing, examination, and inspection functions to be performed, including hold points during fabrication and assembly, as well as during the factory acceptance testing (FAT).

All inspection and testing functions shall be performed by qualified personnel using qualified procedures in accordance with the design and code of record(s).

Hold points are required during the fabrication process to allow inspection, verification, or approval by LANL before the Subcontractor does further work. Hold points shall be identified on the Test and Inspection Plan, with provisions for LANL review and acceptance. LANL has the right to waive Hold Points at their discretion.

Examination Records: The Subcontractor shall appropriately record, submit and maintain records documenting the examination and/or test then submit the completed record as part of the QA Document Package. The status of all planned and executed examinations, inspections, and testing activities shall be logged and traceable to ensure that the required examinations, inspections, and testing have been performed, and any items that have failed inspection or testing are not inadvertently installed or implemented.

Qualifications

Qualification of Examination Personnel

Personnel performing examination shall be qualified in accordance with Subcontractor’s written practice.

Qualification and Certification of examination personnel shall be maintained and available for Inspector’s review.

Examination records including examiner’s qualifications, procedures, and reports shall be retained for at least 5 years.

Owner’s Inspector and Designee shall be qualified in accordance with <<ASME B31.3>> Section 340 and LANL’s Engineering Services Division will act for DOE to designate Owner’s Inspectors or Designee.

      - ***************************************************************************************

      - Items in PART 2 may be excluded in this Section if included in piping specification Sections in Div 02 – 48. Note and address the following:

        - 1. For projects with multiple flange sizes, it is suggested to include a table with information on flanges, fasteners, washers etc. in a bill of materials in the drawings or piping specification sections in Div 02 – 48. Alternatively, each joint may be specified using a design data sheet. An example of a design data sheet is available in Appendix A, Example of Alternative Gasket/Bolting Design Data Sheet.

        - 2. Gasket materials shall be compatible with the fluids and service conditions.

        - 3. Fasteners are to be in accordance with ASME B1.1 UNS Classes 1A (external) and 1B (internal) uncoated unless otherwise specified.

        - 4. Bolt and nut material grades shall be in compliance with the ASME B16.5 Table 1B recommendations.

        - 5. Comply with ASME B16.20 for metallic gaskets, and ASME B16.21 for non-metallic gaskets.

        - 6. Flange surface finish (roughness) shall be compatible with the gasket material. Soft gaskets such as neoprene should have a rough finish (e.g., 250AA) and flat metal gaskets should have a smoother finish (e.g., 32AA); see ASME PCC-1 Table C-1.

        - 7. Specified fasteners must comply with the recommendations of ASME B16.5 and gasket type.

      - ***************************************************************************************

Part 2 - PRODUCTS

product options and substitutions

Alternate products may be accepted per Section << 01 2500 - Substitution Procedures>>.

      - ***************************************************************************************

      - The following Articles are designer choices based on project-specific conditions.

      - ***************************************************************************************

Flange Joint

<<ASME B16.5>>​ Flange:

<< ASME B31.1;  ASME B31.3  Category D;  ASME B31.3  Category Normal;  ASME B31.5;  ASME B31.8;  ASME B31.9; or  NFPA 54>>​.

Fluid: ​​<< Hydronics;  RLW;  glycol; or  _______>>​​.

Material Group: ​<< 2.3; or  _______>>​.

Material Specification: << ASTM A182/A182M Gr. F316L; or  _______>>.

Class: ​<< 150; or  _______>>​.

Size: ​​<< 1;  2;  4; or  _______>>​​​ inch

Type: ​<< full face raised face, ring joint; or  serrated concentric, serrated spiral finish>>​.

Flange finish (Ra): ​<< 250 micro inches (general use with flange gasket spiral-wound, corrugated metal jacket with corrugated metal core, grooved metal gasket with facing layer such as graphite or PTFE (polytetrafluoroethylene (e.g., Teflon ©)); or  125 micro inches (dangerous liquids; any gases>>​.

Flange circumferential flatness: variation less than ​<< 0.006 in hard gasket, 0.01 in soft gasket; or  _______>>​.

Flange radial flatness: variation less than ​<< 0.006 in hard gasket, 0.01 in soft gasket; or  _______>>​.

        - ******************************************************************************

        - For heat exchanger-type application, define the maximum acceptable pass-partition surface height versus flange face; see ASME PCC-1 Table D-1.

        - ******************************************************************************

Manufacturer: ​<< FNW; or  _______>>​.

Part Number: ​<< FNW B7 D052 L0350; or  _______>>​.

      - ***************************************************************************************

      - Fasteners shall comply with ASME B16.5 (latest edition or successor document), Table 1B–List of Bolting Specifications and Table 1C–Flange Bolt Dimensional Requirements. For example, ASME B16.5 fasteners are shown in Table A1. If using other flange standards, follow fastener requirements in those standards.

      - Table A1 - Flange Fasteners

| Nominal Pipe Size (NPS) | Class 150 |  | Class 300 |  | Class 600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bolts1 | Size | Bolts1 | Size | Bolts1 | Size |
| ½ | 4 | ½” | 4 | ½” | 4 | ½” |
| ¾ | 4 | ½” | 4 | 5/8” | 4 | 5/8” |
| 1 | 4 | ½” | 4 | 5/8” | 4 | 5/8” |
| 1 ½ | 4 | ½” | 4 | ¾” | 4 | ¾” |
| 2 | 4 | 5/8” | 8 | 5/8” | 8 | 5/8” |
| 2 ½ | 4 | 5/8” | 8 | ¾” | 8 | ¾” |
| 3 | 4 | 5/8” | 8 | ¾” | 8 | ¾” |
| 4 | 8 | 5/8” | 8 | ¾” | 8 | 7/8” |
| 6 | 8 | ¾” | 12 | ¾” | 12 | 1” |
| 8 | 8 | ¾” | 12 | 7/8” | 12 | 1 1/8” |
| 10 | 12 | 7/8” | 16 | 1” | 16 | 1 ¼” |
| 12 | 12 | 7/8” | 16 | 1 1/8” | 20 | 1 ¼” |
| 14 | 12 | 1” | 20 | 1 1/8” | 20 | 1 3/8” |
| 16 | 16 | 1” | 20 | 1 ¼” | 20 | 1 ½” |
| 18 | 16 | 1 1/8” | 24 | 1 ¼” | 20 | 1 5/8” |
| 20 | 20 | 1 1/8” | 24 | 1 ¼” | 24 | 1 5/8” |
| 24 | 20 | 1 ¼” | 24 | 1 ½” | 24 | 1 7/8” |

      - Notes: 1. “Bolts” refer to the quantity of all types of threaded fasteners used in flange joint assembly (e.g., bolts or studs).

      - ***************************************************************************************

Externally threaded fasteners

Material Specification: ​<< ASTM A193/A193M; or  _______>>​.

Grade: ​​<< B7 (high strength); or  _______>>​​.

Size: ​​<< 5/8 inch diameter x 3.5 inch long stud bolt (minimum);  5/8 bolt diameter 3.00 minimum length machine bolt; or  _______>>​​.

Type: <<ASME B18.2.1>>, ​​<< heavy hex head; or  square head>>​​.

Thread: <<ASME B1.1>>, ​​<< 2A Coarse; or  >1 inch eight thread series>>​​.

Plating: ​​<< None (light oil coat); or  _______>>​​.

Quantity per Flange: ​​​<< 4;  6;  8;  12;  16;  20;  24; or  _______>>​​​.

Manufacturer: ​​<< FNW; or  _______>>​​.

Part Number: ​​<< FNW B7 D052 L0350; or  _______>>​​.

Internally threaded fasteners

Material Specification: ​<< ASTM A194/A194M; or  _______>>​.

Grade: ​<< 2H (high strength); or  _______>>​.

Size: ​<< 5/8 inch; or  _______>>​.

Type: <<ASME B18.2.2>>, ​<< heavy hex head; or  square head>>​.

Thread: <<ASME B1.1>>  ​<< 2B Coarse; or  >1 inch eight thread series>>​.

Plating: ​<< None (light oil coat); or  _______>>​.

Quantity per Flange: ​​<< 4;  6;  8;  12;  16;  20;  24; or  _______>>​​.

Manufacturer: ​<< FNW; or  _______>>​.

Part Number: ​<< FNW A194 H2; or  _______>>​.

Washers

Material Specification: ​<< ASTM F436/F436M; or  _______>>​.

Material Type: ​<< 3 (weathering steel) through-hardened, surface HRC 38 to 45, minimum core HRC 30; or  _______>>​.

Style: Circular

Size: ​<< 5/8 inch; or  _______>>​.

Plating: ​<< None (light oil coat); or  _______>>​.

Quantity per Flange: ​​<< 4;  6;  8;  12;  16;  20;  24; or  _______>>​​.

Manufacturer: ​<< FNW; or  _______>>​.

Part Number: ​<< FNW A194 H2; or  _______>>​.

Metallic Gaskets

Material Specification: <<ASME B16.20>>.

Fluid Service: ​​​<< Category Normal; or  Category D>>​​​.

Fluid: ​​<< RLW;  Compressed Gas less than 150 psig; or  _______>>​​.

Type: ​​<< Spiral-Wound Gaskets (SWG) 316SS, flexible graphite filter GRAFLEX®, green edge denoting 316; or  _______>>​​.

Pressure Rating: ​​<< Per ASME B16.5; or  _______>>​​.

Temperature Rating: ​<< 32 to 842 degrees Fahrenheit; or  _______>>​.

Size: ​<< 4 inch flange, 1/8 inch thickness; or  _______>>​.

Manufacturer: ​<< FNW; or  _______>>​.

Part Number: ​<< FNW SWG 16FP; or  _______>>​.

Non-Metallic Gaskets

Material Specification: <<ASME B16.21>>.

Fluid Service: ​​​<< Category Normal; or  Category D>>​​​.

Fluid: ​​<< RLW;  Compressed Gas less than 150 psig; or  _______>>​​.

Type: ​​<< Red Rubber Gaskets; Styrene-Butadiene (SRB), 75 Shore A +/- 5; or  _______>>​​.

Pressure Rating: ​<< 200 psig; or  _______>>​.

Temperature Rating: ​<< -20 to 170 degrees Fahrenheit; or  _______>>​.

Size: ​<< 4-inch flange, 1/8 inch thickness; or  _______>>​.

Manufacturer: ​<< FNW; or  _______>>​.

Part Number: ​<< FNW R 1FFGAP; or  _______>>​.

Electrical Isolation Kit

Fluid Service: ​​​<< Category Normal; or  Category D>>​​​.

Size: ​<< 4-inch flange, 1/8 inch thickness; or  _______>>​.

Flange Type:  << Full face flange;  Raised faced flange; or  Ring flange>>.

Gasket Type: ​​<< E;   F; or  D>>​​ (matches flange type above respectively).

Gasket Sealing Element, Sleeve, and Washer Material: ​<< None; or  Black polyethylene plastic>>​.

Manufacturer: ​<< M&P; or  Flange and Pipe Protection, Inc.>>​.

Fastener Assembly Lubricant

      - ***************************************************************************************

      - The following two paragraphs are designer choices.

      - ***************************************************************************************

Stainless Steel

Rocol Accu-Lube 14143 (low or no halide content)

Supplier: Aviall Services INC, Dallas, TX 1-800-284-2551

Carbon Steel

Loctite 51606 C-100 Anti-Seize Lubricant

Supplier: Diamond Tool, 888-879-3426

Gasket Spray Adhesive

3M Hi-Strength 90 Spray Adhesive

PART 3 - EXECUTION

    - **********************************************************************************************

      - Fasteners shall be segregated to ensure the correct fastener is used on the correct application.

      - Torque requirements for flange connections shall be in accordance with the gasket manufacturer’s torque requirements and the recommendations of ASME B16.5.

      - Gaskets shall be segregated to ensure the correct gasket is used on the correct application.

    - **********************************************************************************************

GENERAL NOTES

Piping (and tubing) Systems: Fabricate, inspect, examine, and test in accordance with <<ASME B31.3>>.

Remove scale, slag, dirt, and debris from inside and outside of piping components.

PREPARATION

      - ***************************************************************************************

      - Review LANL ESM Chapter 17 - Pressure Safety, Sections PS-REQUIREMENTS and PS-GUIDE for information on cleaning.

      - ***************************************************************************************

Pipe Cleaning

Piping shall be cleaned in accordance with << define required cleaning process>> and meet the cleaning specified requirement << define the state of the item after cleaning: particulate size and counts, amount and type of non-volatile residue, unacceptable or acceptable chemical residue, or biologically sterile>>.

Subcontractor shall be responsible for the cleanliness and integrity of the system. Flange mating surfaces shall be free of dirt, paint, metal chips, filings, flux, slag, weld spatter, scale, rust, grease, oil, waxes, or other detrimental material that are easily seen with the unaided eye.

Consult Manufacturer’s recommendation for the use of acids and cleaning agents to prevent damage. Cleaning agents used with stainless steel systems shall contain no more than 50 ppm halide content.

Ensure safeguards are taken to protect personnel from hazards of cleaning, which may include but not be limited to flying particulates, corrosive chemicals, and harmful vapors.

A suitable chemical or mechanical cleaning method shall be used, if necessary, to clean all surfaces to be fabricated.

A soft wire brush of the same material type as the flange is required. For example, austenitic stainless-steel brushes are required for F316L flanges.

PRE-ASSEMBLY

General

Verify materials are correct and properly segregated before assembly in accordance with the accepted Material Control Procedure.

Fasteners

Fastener materials shall be free of nicks, burrs, chips, dirt, and damage (inspect threads, shank, and nuts). All damaged fasteners must be replaced.

For new, coated bolts and nuts, nuts must run freely by hand past where they will come to rest after tightening. If nuts will not turn freely by hand, check for cause and make necessary corrections/replacements. Lubricant application is limited to the second and subsequent tightening operations since the coating provides sufficient lubrication for the first tightening.

For non-coated bolts or second and subsequent tightening of all bolts, apply lubricant liberally and completely to the nut contact faces and to the threads on both ends of the bolts past where the nuts will come to rest after tightening; the lubricant should be applied after the bolts are inserted through the flange bolt holes to avoid possible contamination with solid particles that could create unwanted reaction torque.

Flanges

Visually inspect the face surface finish of all flanges for injurious defects. Surface shall be free of any defects, nicks, or burrs.

Faces (both raised and flat) shall be clean, free of debris or foreign material and degreased. Imperfections such as nicks, dents or gouges will affect the gasket’s ability to seal.

All rust and burrs must be removed, and small scratches should be removed by polishing; other defects may mean component replacement.

Permissible imperfections in flange surface finish are specified in Table 1.

        - ******************************************************************************

        - Table 1 shall be checked for accuracy per applicable edition of ASME B16.5 or successor document. ASME PCC-1 Table D-2 may be used as an alternative to Table 1 Permissible Imperfections in Face Finish shown below, which is based on ASME B16.5 Table II-3.

        - ******************************************************************************

        - Table 1 - Permissible Imperfections in Face Finish

| ​ Nominal Pipe Size (NPS) | ​Maximum Radial Projection of Imperfections Which are No Deeper than the Bottom of the Serrations (inch) | ​ Maximum Depth and Radial Projection of Imperfections Which are Deeper than the Bottom of the Serrations (inch) |
| --- | --- | --- |
| ½ through 2½ | 0.12 | 0.06 |
| 3 | 0.18 | 0.06 |
| 4 through 6 | 0.25 | 0.12 |
| 8 through 14 | 0.31 | 0.18 |
| 16 | 0.38 | 0.18 |
| 18 through 24 | 0.50 | 0.25 |

        - ******************************************************************************

        - Refer to ASME PCC-1 Table D-1 for flange seating face flatness tolerances.

        - ******************************************************************************

Check raised-face and flat-face flanges for flatness with a straight edge.

Check the spiral (phonographic) or concentric groove on the face of raised- and flat-face flanges. Any radial defects for example will be virtually impossible to seal against; replace as necessary.

Ring-type-joint (RTJ) grooves must be kept scrupulously clean, corrosion free, and undamaged.

When cleaning a flange face, never use a tool that may damage the surface finish.

Correct out-of-tolerance conditions before the gasket is installed to avoid damage to the gasket. Only minimum or reasonable adjustments may be made after the gasket is installed.

Alignment

When aligning requires more force than can be exerted by hand or common hand-and-hammer alignment tools such as spud wrenches and alignment pins, consult the LANL STR. A detailed pipe stress analysis may be necessary to confirm that the system can withstand the additional forces from other alignment methods used.

If the flanges in need of alignment are connected to pumps or rotating equipment, great care must be taken to prevent introducing a strain into the equipment housing or bearings. Mark the equipment position prior to assembly and measure the movement after installation is complete. Measuring the movement in the equipment to ensure that its aligned condition is not disturbed is a common and necessary practice. Any distortion of piping to bring it into alignment for joint assembly that introduces a detrimental strain in equipment or piping components is prohibited.

Align piping and vessel flanges so that there are equal distances between the flange faces at all points around the circumference of the joint, therefore making the flange faces parallel to each other. The tolerance is usually determined by measuring the closest and farthest distance between the flanges and comparing. An acceptable practice is a difference no greater than 0.8 mm (1/32 inch) at the O.D. of the sealing surface, achieved using a force of no greater than 10% of the maximum torque or bolt load for any bolt.

Align piping or vessel flanges so that the bolt holes align with each other, allowing the fasteners to pass through at right angles to the flanges and the nuts resting flat against the flanges prior to tightening. The tolerance is measured by observing a 90 degree angle where the fastener passes through the flanges or the holes are within 3 mm (1/8 inch) of perfect alignment.

        - ******************************************************************************

        - The following two subparagraphs are designer choices.

        - ******************************************************************************

Prior to final assembly, flange faces shall be aligned properly. Unless otherwise approved by engineering, the bolt holes shall be (1) aligned within ​<< _______>>​ inch maximum offset and (2) the flange faces be parallel to within ​<< _______>>​ in/ft. measured across any diameter; both are illustrated in Figure 1. For parallelism, the maximum allowable gap difference across the face when aligning ​<< Class _______; or  _______>>​ Flanges is ​<< _______>>​ in.

Prior to final assembly, flange faces shall be aligned properly. Unless otherwise approved by engineering, the bolt holes shall be (1) aligned within 1/8 inch maximum offset and (2) the flange faces be parallel to within 1/16 in/ft. measured across any diameter; both are illustrated in Figure 1. For parallelism, Table 2 presents the maximum allowable gap difference across the face when aligning Class 150, 300, and 600 Flanges.

        - ******************************************************************************

        - Per ASME B31.3, paragraph 335.1(c)(1) Before bolting, mating gasket contact surfaces shall be aligned to each other within 1 mm in 200 mm (1/16 inch/foot [0.5%]), measured across any diameter.

        - ******************************************************************************

        - Figure 1 – <<ASME B31.3>> Maximum Flange Misalignment Criteria

        - Table 2 – <<ASME B31.3>>​ Maximum Allowable Gap for Class 150, 300, and 600 Flanges

| ​ Nominal Pipe Size (NPS) | ​ Maximum Allowable Gap (inch) |  |  |
| --- | --- | --- | --- |
|  | ​ Class 150 | ​ Class 300 | ​ Class 600 |
| ½ | 0.018 | 0.020 | 0.020 |
| ¾ | 0.020 | 0.024 | 0.024 |
| 1 | 0.022 | 0.025 | 0.025 |
| 1 ½ | 0.026 | 0.032 | 0.032 |
| 2 | 0.031 | 0.034 | 0.034 |
| 3 | 0.039 | 0.043 | 0.043 |
| 4 | 0.049 | 0.052 | 0.056 |
| 6 | 0.057 | 0.065 | 0.073 |
| 8 | 0.070 | 0.078 | 0.086 |
| 10 | 0.083 | 0.091 | 0.104 |
| 12 | 0.099 | 0.107 | 0.115 |

        - ******************************************************************************

        - If approved by the engineering design, heat may be used to relieve the pipe stress, bend the pipe, and aid in piping alignment.

        - ASME B31.3, paragraph 335.1(b) Heating shall not be used to help in closing the gap because it defeats the purpose of cold springing.

        - ******************************************************************************

In joints where one or more of the flanges are not attached to piping or vessels, such as cover plates and tube bundles, use minimum-necessary and reasonable force to accomplish the best aligned condition.

When no external alignment devices are used, the flanges should be brought into contact with the uncompressed gasket uniformly across the flange faces using all the bolts/studs less than the equivalent of 10% of the total target assembly bolt load. When aligning the flanges, no single bolt should be tightened above 20% of the single bolt maximum torque or target bolt load.

When external alignment devices are used, the flanges should be brought to the compressed gasket thickness uniformly across the flange faces using an external load equivalent to less than 20% of the total target assembly bolt load. If more force is required to bring the flange gap into compliance, consult an engineer.

Repair the misaligned component by replacing it, correctly removing and reinstalling it in the properly aligned position<< , or using uniform heat to relieve the stresses; or  None - N/A>>.

Gaskets

No more than one gasket shall be used between contact faces in assembling a flanged joint.

Ensure that gaskets are of the correct material and size. Gaskets must be clean and damage free; not bent, broken, torn, nicked, or distorted in any manner. The allowable shelf life must be checked.

Do not cut or deform a gasket to fit a flange. If the gasket doesn’t fit, the wrong one has been chosen.

FLANGE ASSEMBLY

      - ***************************************************************************************

      - 1. Other, more accurate, methods of obtaining preload such as bolt tensioners or measurement of bolt elongation may be substituted for torque wrenches if performed in accordance with an approved procedure.

      - 2. The proper lubricant shall be used on threads of bolts and nuts. Consult the manufacturer for the recommended lubricant.

      - 3. Where a metallic flange is bolted to a nonmetallic flange, both shall be flat faced. The use of a gasket between these two flanges is not mandatory unless specified by the manufacturer. For nonmetallic flanges bolting torque shall be based on the manufacturer’s recommendations and limited so that the nonmetallic flange is not overloaded. Backup flanges may be necessary for the nonmetallic flange; ASTM A395/A395M, Standard Specification for Ferritic Ductile Iron Pressure-Retaining Castings for Use at Elevated Temperatures.

      - 4. After leak test and system startup, thermal expansion, creep relaxation, and fastener thread embedment can change the fasteners’ applied load. Re-torque after 48 hours is recommended. Therefore, a re-check of each fastener is recommended especially for piping systems that have non-metallic gaskets and operate at temperature levels above 200 degrees Fahrenheit.

      - ***************************************************************************************

All flange joints shall be assembled in accordance with provided drawings.

Special Instructions

When a flanged joint has been made up and subsequently loosened, the joint shall be cleaned, and a new gasket installed, prior to retightening the joint.

Electrical isolation kits shall be used when mating dissimilar metals.

Place a new gasket in position after determining the absence of (or having made correction for) unacceptable gasket sealing surface imperfections and flatness tolerance deviations, as well as joint alignment considerations.

Verify that the gasket complies with the dimensional (O.D., I.D., thickness) and material specifications.

Position the gasket to be concentric with the flange I.D., taking suitable measures to ensure that it is adequately supported during the positioning process. No portion of the gasket should project into the flow path.

        - ******************************************************************************

        - Particular care should be taken to avoid adhesive chemistry that is incompatible with the process fluid or could result in stress corrosion cracking or pitting of the flange surfaces. Allowable and/or prohibited adhesive materials must be identified in the Section.

        - ******************************************************************************

Spray very light dusting of spray adhesive on the gasket (not the flange).

Ensure that the gasket will remain in place during the joint assembly process.

Do not use tape strips radially across the gasket to hold it in position. Do not use grease.

Do not draw the flange up tight on one or two bolts only. This will cause local gasket crushing or pinching and will ultimately result in a leaking flange.

Flange Assembly

Number the bolts/studs and the nuts clockwise as an aid to identification and for applying the criss-cross bolt tightening sequence clockwise on the flange. An example is shown in Figure 2.

        - Figure 2 – 4-Bolt Flange Numbering (Example)

Hand-tighten the stud bolts with a short wrench first before torquing.

        - ******************************************************************************

        - It is recommended to follow the latest edition of ASME PCC-1 or successor document for acceptable sequencing. One example is the tightening sequence for the flange bolts shown below from ASME PCC-1 Table J-6-1.

        - Define the assembly pattern to use in Table 3. Add rows if necessary.

        - ******************************************************************************

The sequence in which bolts are tightened is extremely important. If performed improperly, tightening can cause the flange to move out of parallel. A staggered crisscross-torquing pattern, must be used to tighten the bolts. The tightening sequence shall be as noted in Table 3.

        - Table 3 – Torque Sequence

| Number of Bolts | ​ Bolt Torquing Sequence |
| --- | --- |

Hidden Paragraph

| ​​<< _______>>​​ | ​​<< _______>>​​ |
| --- | --- |

Hidden Paragraph

| ​​<< _______>>​​ | ​​<< _______>>​​ |
| --- | --- |

Hidden Paragraph

| ​​<< _______>>​​ | ​​<< _______>>​​ |
| --- | --- |

Hidden Paragraph

| ​<< _______>>​ | ​<< _______>>​ |
| --- | --- |

Insert the bolt/stud through the flange and, if necessary, lubricate the threads (see Article Pre-Assembly).

Install hardened-steel washers and/or lubricate smooth face of nut (i.e., face under compression load). Raised lettering on the nuts must be facing outwards.

Use only calibrated torque wrenches and multipliers. Examine the torque wrench for proper calibration, damage, and proper range. Torque wrenches can be used only within the range specified by the wrench manufacturer. Specified working ranges typically vary from 25% - 75% to 20% - 100% of the full torque wrench range.

Appropriate calculations must be made when using multipliers, e.g., a crow’s foot, or any other attachment which adds length to the torque wrench (cheater bars are not acceptable). Consult an engineer for adjustment to the appropriate torque setting.

Prior to bolting up, flange faces shall be aligned properly. Ideally, the flange faces should be parallel to within 1/16 inch/foot (0.5%) measured across any diameter and the bolt holes shall be aligned within 1/8 inch maximum offset (refer to Figure 1 above).

Required Torque

Torque tolerances

+/- 2 ft-lb if torque value is less than 50 ft-lb

+/- 4 ft-lb if torque value is greater than 50 less than 100 ft-lb

+/- 4% of torque value if greater than 100 ft-lb.

The following Table 4 shall be the final torque values with torque increments applied to the flange joints required in this design.

        - ******************************************************************************

        - Note: Only one flange size with two different gaskets is shown in Table A1 - Final Torque Values (Example) below. However, Table 4 shall contain all the flange torques required for the design. Designer may add additional rows to Table 4 if necessary. ASME PCC-1 Appendix O Assembly Bolt Stress Determination can be used to calculate the bolt torque requirements.

        - In addition to initial compression, a residual compression value, after internal pressure is applied, is required to maintain the seal. A minimum residual gasket compression of 4 to 6 times the working pressure is standard practice.

        - Table A2 – Final Torque Values (Example)

| ​ Flange Size | ​ Gasket | ​ Uncoated Bolts Final Torque (ft-lb) | ​ Coated Bolts Final Torque (ft-lb) | ​ Lubricated Bolts Final Torque (ft-lb) |
| --- | --- | --- | --- | --- |
| 4 inch | Spiral-wound (SWG) | 145 | 120 | 120 |
| 4 inch | Rubber (SBR) (styrene-butadiene rubber) | 80 | 66 | 66 |

        - ASME PCC-1:

        - “It is recognized by Appendix S of the ASME Boiler and Pressure Vessel Code, Section VIII, Division 1 that the initial tightening of the bolts in a joint comprising flanges designed in accordance with Appendix 2 of that Code is a prestressing operation and that the level of required Target Bolt Prestress can vary considerably above the code tabulated design-stress value. This is an acceptable and usually required practice. Appendix S states that “. . . an initial bolt stress higher than the design value may and, in some cases, must be developed in the tightening operation, and it is the intent of this Division that such a practice is permissible, provided it includes necessary and appropriate provision to ensure against excessive flange distortion and gross crushing of the gasket.” For joints custom designed in accordance with Appendix 2, a common range of Target Bolt Prestress that is often found acceptable is around 40% to 70% of the specified minimum yield strength of the bolt material. This range is normally only exceeded in exceptional cases that have been assessed by a qualified engineer. However, any maximum Target Bolt Prestress must be selected to ensure that all three of the joint components—bolts, flange, and gasket—are stressed within acceptable limits.”

        - ******************************************************************************

        - Table 4 – Final Torque Values with Torque Increments

| ​ Flange Size | ​ Flange Class | ​ Gasket | ​ Bolt Specification | ​ << Uncoated; Coated; or Lubricated>> Bolts Final Torque (ft-lb) | ​ Torque Increments (ft-lb) |
| --- | --- | --- | --- | --- | --- |

Hidden Paragraph

| << 4 inch; or ______>> | << ____>> | << Spiral wound (SWG); or ___>> | << ____>> | << 120; or ___>> | << 40, 80, 120; or _______>> |
| --- | --- | --- | --- | --- | --- |

Hidden Paragraph

| << 4 inch; or ______>> | ​​<< ____>>​​​ | ​​<< Rubber (SBR); or ______>>​​​ | ​​<< ____>>​​ | ​​<< 66; or ______>>​​ | ​​<< 22, 44, 66; or _______>>​​​​ |
| --- | --- | --- | --- | --- | --- |

Hidden Paragraph

| ​​​​​​<< ____>>​​​​​​ | ​​​​​​<< ____>>​​​​​​ | ​​​​​​<< ____>>​​​​​​ | ​​​​​​<< ______>>​​​​​​ | ​​​​​<< ______>>​​​​​ | ​​​​​<< ______>>​​​​​ |
| --- | --- | --- | --- | --- | --- |

Hidden Paragraph

| ​​<< ____>>​​ | ​​<< ____>>​​ | ​​<< ____>>​​ | ​​<< ______>>​​ | ​<< ______>>​ | ​<< ______>>​ |
| --- | --- | --- | --- | --- | --- |

Hidden Paragraph

| ​<< ____>>​ | ​<< ____>>​ | ​<< ____>>​ | ​<< ______>>​ | << ______>> | << ______>> |
| --- | --- | --- | --- | --- | --- |

Tightening shall be done in a minimum of three increments (1/3 of required torque) to reach final torque. No more than one third of the final torque should be achieved during a single step (refer to Table 4).

Following the first torquing pass confirm that the flanges are parallel. If flanges are not parallel at this point, make two complete passes around the flange at 1/3 of the specified final torque to equalize load in the fasteners and examine again for parallelism. If the flanges are not parallel and full-face contact has not been achieved, engineering must evaluate for excessive misalignment.

Post-Assembly

When sequential torquing is complete, use rotational and reverse rotational tightening to check that all nuts are stable. Verify nuts are stable when the torque wrench does not turn before the wrench achieves the final bolt torque value.

Nuts shall have full thread engagement on the bolts or studs. One to two exposed threads is the preferable requirement that defines full thread engagement. The minimum acceptable engagement is the outer edge of the nut being not less than flush with the end of the bolt or stud.

Visually check to ensure against excessive flange distortion and gross crushing of the gasket.

Check the flange face-to-face separation once the gasket has been installed and the bolts tensioned. A uniform separation is required. If not, the gasket could be locally crushed or deformed and will not seal properly.

Re-torque after approximately 24 hours following initial assembly, and prior to any leak or pressure tests.

EXAMINATION, TESTING, AND INSPECTION

For the purposes of this section, the Subcontractor is responsible for all tasks identified as examination. Owner’s Inspector activities are considered inspection and is provided by LANL personnel.

Examination

Both the extent of examination and acceptance criteria shall be in accordance with <<ASME B31.3>> (for example paragraph 341 Examination and/or, for non-metallic systems, paragraph A341 as applicable.)

When pneumatic leak testing is planned, 100% of all threaded, bolted, and other mechanical joints shall be examined during assembly.

Testing

For gas leak testing it is common practice to leak test flange joints by carefully wrapping and sealing the joint with tape, punching a small hole in the tape and applying leak test solution to the hole. Any bubbling thus indicates leakage at the flange joint. Pressure-sensitive masking tape or cloth-backed, pressure-sensitive tapes of either the waterproof or water-vapor-resistant types may be used.

See Div 02-48 for specific testing requirements for piping systems.

Inspection

Owner’s Inspector shall have access to any and all design, fabrication, manufacture, fabrication, heat treatment, assembly, erection, examination, testing, records, documentation, or other project information or activities to verify that all required examinations and testing have been completed and to inspect the piping to the extent necessary to be satisfied that it conforms to all applicable examination requirements of the Code and of the engineering design and to perform the role defined in <<ASME B31.3>>.

Owner’s Inspector is the final authority on acceptance of the project, examination, or test.

END OF MAIN SECTION​​​​​​<< ; APPENDIX FOLLOWS; or  None - N/A>>​​​​​​

  - End of Section

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 01 4731, REV. 3, DATED APRIL 15, 2026.

APPENDIX A

Example of Alternative Gasket/Bolting Design Data Sheet

END OF SECTION