Section 08 1400 
Wood Doors

  - *************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 6 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - *************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

Interior Flush Wood Doors and Frames - ​​​<< Click to select fire-rated/non-fire rated option(s) from drop down menu;  Fire-rated; and  Non-fire rated>>​​​.

RELATED REQUIREMENTS

Section << 01 3300 - Submittal Procedures>>

Section << 01 8113.13 - Sustainable Design>>

Section << 08 1213 - Hollow Metal Frames>>

Section << 08 7100 - Door Hardware>>

Section << 08 8000 - Glazing>>

REFERENCE STANDARDS

AWI (AWS) - Architectural Woodwork Standards.

BAAQMD 8-51 - Bay Area Air Quality Management District Regulation 8, Rule 51, Adhesive and Sealant Products.

CAL (CDPH SM) - Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers Version 1.2.

CARB (ATCM) - Airborne Toxic Control Measure to Reduce Formaldehyde Emissions from Composite Wood Products.

FSC Guidelines - Forest Stewardship Council Guidelines.

FSC-STD-01-001 - FSC Principles and Criteria for Forest Stewardship.

GreenSeal GS-36 - Standard for Adhesives for Commercial Use.

NFPA 80 - Standard for Fire Doors and Other Opening Protectives.

NFPA 252 - Standard Methods of Fire Tests of Door Assemblies.

SCAQMD 1168 - Adhesive and Sealant Applications.

TSCA - Toxic Substances Control Act.

UL (DIR) - Online Certifications Directory.

USGBC LEED v4-BD+C - LEED v4 for Building Design and Construction.

WDMA I.S. 1A - Interior Architectural Wood Flush Doors.

      - ***************************************************************************************

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

SUBMITTALS

Submit the following in accordance with the requirements of Section << 01 3300 - Submittal Procedures>>.

Action Submittals:

Catalog data for each type of wood door, including details of core and edge construction, trim for openings, and finishing specifications for doors to receive factory finish.

        - ******************************************************************************

        - Retain the following subparagraph when project requires samples.

        - ******************************************************************************

Color and finish samples of factory finished doors.

Shop drawings indicating location, size, and hand of each door, elevation of each kind of door, details of construction, location and extent of hardware blocking, fire ratings, requirements for factory finishing and other pertinent data.

        - ******************************************************************************

        - Verify these requirements below against LANL ESM Chapter 14 - Sustainable Design, Attachment 1 "Required Attribute" Column and, if applicable, the LEED prerequisites and project credits. Material content and product data for recycled content are required. The remaining submittals are designer-choices based on project-specific requirements.

        - ******************************************************************************

Sustainable Design Submittals:

Sustainable Design (SD) Submittals: Provide material content documentation in accordance with the requirements of this Section. ​​​​​​​<< Submit any additional documentation for this product category, as applicable under Section  01 8113.13 - Sustainable Design , with this Section’s submittal.; or  None - N/A>>​​​​​​​

          - **********************************************************************

          - Green building rating systems typically include credit for sustainably harvested wood. USGBC-LEED™, for example, includes credit for use of sustainably harvested wood certified under Forest Stewardship Council Guidelines.  Under LEED™ v3, a minimum of 50 percent of wood-based materials and products incorporated into the Project must be certified in accordance with the Forest Stewardship Council Guidelines. Under LEED™ v4, products meeting wood products criteria are valued at 100% of their cost for the purposes of credit achievement calculation.

          - **********************************************************************

Chain-of-custody certificates indicating if products comply with forest certification requirements. Include documentation that manufacturer is certified for chain of custody by an FSC-accredited certification body. Include statement indicating cost for each certified wood product.

Forest Stewardship Council (FSC): Provide letter of certification signed by lumber supplier. Indicate compliance with <<FSC-STD-01-001>> and identify certifying organization.

Submit FSC certification numbers; identify each certified product on a line-item basis.

          - **********************************************************************

          - Green building rating systems may include credit for low emitting materials. USGBC-LEED™ v3, for example, includes credits for low-emitting materials, including: adhesives and sealants, paints and coatings, carpets, and composite wood and agrifiber products. Under LEED™ v4, adhesives and sealants are to comply with California’s South Coast Air Quality Management District (SCAQMD) #1168 and aerosol adhesives are to comply with Green Seal GS-36.

          - **********************************************************************

Engineered Wood Products: Provide documentation that composite wood and agrifiber products are in compliance with the <<TSCA>> 1976’s amendment from 2016, Subchapter VI – Formaldehyde Standards for Composite Wood Products.

Product Data for adhesives, documentation including printed statement of VOC content.

            - *************************************************************

            - Retain the first subparagraph for aerosol adhesives. Retain the second subparagraph for adhesives and sealants. Selecting adhesive type here updates the entire Section.

            - *************************************************************

For Aerosol Adhesives: Submit Green Seal Certification to <<GreenSeal GS-36>> and description of the basis of certification.

For Adhesives and Sealants: Submit manufacturer’s certification that products comply with <<SCAQMD 1168>> in areas where exposure to freeze/thaw conditions and direct exposure to moisture will not occur. In areas where freeze/thaw conditions do exist or direct exposure to moisture can occur, submit manufacturer’s certification that products comply with <<BAAQMD 8-51>> for containers larger than 16 oz and with <<CARB (ATCM)>> for containers 16 oz or less.

          - **********************************************************************

          - LEED grants one credit to a project for using materials with recycled content that is the sum of postconsumer recycled content plus one-half the preconsumer recycled content, based on cost.

          - **********************************************************************

Product data for recycled content.

Particleboard/fiberboard

Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

          - **********************************************************************

          - Retain the following subparagraph when providing doors having STC ratings.

          - **********************************************************************

Product data for USDA BioPreferred Products: Indicate percentage of biobased content in the following products:

Composite Panels

Acoustical Panels

Description: Engineered products designed for use as structural and sound deadening material suitable for office partitions and doors.

Recommended Biobased Content: 37 percent minimum.

Laboratory Test Reports for ​<< adhesives; and  composite-wood products>>​ documentation indicating that products comply with the testing and product requirements of <<CAL (CDPH SM)>>.

Informational Submittals:

Warranty as specified.

QUALITY ASSURANCE

Provide flush wood doors complying with the following standards:

<<WDMA I.S. 1A>>​

<<AWI (AWS)>>: Designations for grade and door construction.

      - ***************************************************************************************

      - Retain the following paragraph when fire-rated wood doors are in project-scope.

      - ***************************************************************************************

Fire-Rated Wood Doors: Provide wood doors which are identical in material and construction to units tested in door and frame assemblies per <<NFPA 252>> and which are labeled and listed for ratings indicated by <<UL (DIR)>>, Warnock Hersey or other testing and inspection agency acceptable to Architect of Record.

Manufacturer: Obtain doors from a single manufacturer to ensure uniformity in quality of appearance and construction, unless otherwise indicated.

Provide materials and perform work in accordance with above referenced documents.

      - ***************************************************************************************

      - Verify these requirements below against LANL ESM Chapter 14 - Sustainable Design, Attachment 1 "Required Attribute" Column and, if applicable, the LEED prerequisites and project credits. Material content and product data for recycled content are required. The remaining requirements are designer-choices based on project-specific requirements.

      - ***************************************************************************************

Sustainable Design Requirements

Certified Wood Materials: Furnish wood materials certified in accordance with <<FSC Guidelines>>.

Recovered Content Materials: Furnish materials with recovered content for:

Required: Particleboard/fiberboard component must be 80-100 percent recovered wood or wood composite.

Preferred: 100 percent recovered agricultural fiber.

VOC Emissions: Provide low VOC products.

Adhesives and sealants: Comply with <<SCAQMD 1168>> or with <<BAAQMD 8-51>> for containers larger than 16 oz and with <<CARB (ATCM)>> for containers 16 oz or less.

Aerosol adhesives: Comply with <<GreenSeal GS-36>>.

Engineered Wood Products:

Formaldehyde compliance with the <<TSCA>> 1976’s amendment from 2016, Title VI – Formaldehyde Standards for Composite Wood Products.

VOC emission compliance with the US Green Building Council’s Maximum VOC Concentration Levels outlined in the <<USGBC LEED v4-BD+C>> New Construction Indoor Air Quality Assessment credit.

DELIVERY, STORAGE, AND HANDLING

Protect wood doors during transit, storage, and handling to prevent damage, staining, soiling and deterioration.  Comply with requirements of <<WDMA I.S. 1A>> as well as with manufacturer's instructions. Package doors at factory prior to shipping using manufacturer's standard method.

Identify each door with individual opening numbers which correlate with designation system used on shop drawings for door, frames, and hardware, using temporary, removable, or concealed markings.

WARRANTY

Door Manufacturer's Warranty:  Submit written agreement on door manufacturer's standard form signed by Manufacturer, Installer, and Subcontractor, agreeing to repair or replace defective doors which have warped (bow, cup, or twist) or which show telegraphing of core construction in face veneers, or do not conform to tolerance limitations of <<WDMA I.S. 1A>> and <<AWI (AWS)>>.

Warranty (to become effective after date of final acceptance):

Ten years for solid core flush interior doors.

Replace doors where Subcontractor's work contributed to rejection or to voiding of manufacturer's warranty.

PART 2 - PRODUCTS

MANUFACTURERS

Timberland Door.

VT Industries.

Assa Abloy Doors.

Mohawk Doors.

Marshfield DoorSystems.

<< _______>>.

INTERIOR FLUSH WOOD DOORS AND FRAMES

      - ***************************************************************************************

      - The following two paragraphs are designer-choices. Retain the first paragraph when non fire-rated doors are in project-scope. Retain the second paragraph when fire-rated doors are in project-scope.

      - ***************************************************************************************

Non Fire-Rated Solid Core Doors with Wood Veneer Faces: Comply with the following requirements:

Faces and AWI Grade: 1/8 inch, 3-ply panel, << plain sliced,;  _______ ,; or  None - N/A>><< Premium; or   Custom>> grade, red oak veneer.

Finish: AWI System TR-4.

Core Construction: PC-HPDL (Particleboard core).

Fire-Rated Solid Core Doors: Comply with the following requirements:

Faces and AWI Grade: Provide faces and grades to match non-fire rated doors in same area of building.

Core Construction: Manufacturer's standard core construction as required to provide fire-resistance indicated.

Edge Construction: Provide manufacturer's standard edge construction for improved screw-holding capability and split resistance. Do not use edges composed of a single layer of treated lumber.

Finish: << Fire-retardant Wood Veneer;  Fire-rated Paint/Coating; or  _______>>.

Hollow Metal Frames: Refer to Section ​<< 08 1213 - Hollow Metal Frames>>​

      - ***************************************************************************************

      - Retain the following Article Lite Frames when lite openings are in project-scope.

      - ***************************************************************************************

LITE FRAMES

Frames for Lite Openings: Manufacturer's standard frame formed of 18 gauge cold-rolled steel, factory-primed, and approved for use in door of fire-rating indicated.

For glazing, refer to Section << 08 8000 - Glazing>>.

      - ***************************************************************************************

      - Retain the following Article General Fabrication Requirements when lite openings are in project-scope.

      - ***************************************************************************************

GENERAL FABRICATION REQUIREMENTS

Openings: Cut and trim openings through doors and panels as shown on drawings. Comply with applicable requirements of <<WDMA I.S. 1A>> and <<AWI (AWS)>> for types of doors required.

Lite Openings: Factory cut openings.

          - **********************************************************************

          - Retain the following subparagraph when non-fire rated doors with openings are in project-scope.

          - **********************************************************************

Trim openings for non-fire rated doors with solid wood moldings.

REFITTING AND PREPARATION FOR HARDWARE

Refer to Section << 08 7100 - Door Hardware>> for door hardware for wood doors.

Comply with tolerance requirements of <<AWI (AWS)>> for pre-fitting. Machine doors for hardware requiring cutting of doors. Comply with final hardware schedules and door frame shop drawings and with hardware templates and other essential information required to ensure proper fit of doors and hardware.

Take accurate field measurements of hardware mortises in door frames to verify dimensions and alignment before proceeding with machining in factory.

PART 3 - EXECUTION

INSPECTION

Examine door frames, after their installation, and doors, prior to their hanging, as follows:

Verify that frames comply with indicated requirements for type, size, location, and swing characteristics and have been installed with plumb jambs and level heads.

Verify that doors are free of defects that could cause their rejection.

INSTALLATION

Install hollow metal frames per Section << 08 1213 - Hollow Metal Frames>>.

Condition doors to average prevailing humidity in installation area prior to hanging.

Install wood doors to comply with manufacturer's instructions and <<AWI (AWS)>><< and as indicated; or  None - N/A>>.

        - ******************************************************************************

        - Retain the following subparagraph when fire-rated doors are in project-scope.

        - ******************************************************************************

Install fire-rated doors in corresponding fire-rated frames in accordance with requirements of <<NFPA 80>>.

      - ***************************************************************************************

      - Select one of the following two paragraphs based on project-specific requirements. Retain the first paragraph for job-fit doors. Retain the second paragraph for pre-fit doors.

      - ***************************************************************************************

Job-fit Doors: Align and fit doors in frames with uniform clearances and bevels as indicated below. Machine doors for hardware. Seal cut surfaces after fitting and machining.

Fitting Clearances:

          - **********************************************************************

          - Retain the following subparagraph(s) based on the type of door(s) in project-scope (non-fire rated vs. fire-rated).

          - **********************************************************************

For non-fire rated doors, provide clearances of 1/8 inch at jambs and heads; 1/16 inch per leaf at meeting stiles for pairs of doors; and 1/8 inch from bottom of door to top of decorative floor finish or covering.

For fire-rated doors, provide clearances complying with <<NFPA 80>>.

Bevel:

          - **********************************************************************

          - Retain the following subparagraph(s) based on the type of door(s) in project-scope (non-fire rated vs. fire-rated).

          - **********************************************************************

Non-fire rated doors: 1/8 inch in 2 inch at lock edges.

Fire-rated doors: 1/8 inch in 2 inch at lock edges.

      - *** OR ***

Pre-fit Doors: Fit to frames and machine for hardware to whatever extent not previously worked at factory as required for fit and uniform clearance at each edge.

      - ***************************************************************************************

      - Retain the following paragraph when glazing required for project.

      - ***************************************************************************************

For installation of glazing, refer to Section ​​<< 08 8000 - Glazing>>​​.

ADJUSTMENT AND PROTECTION

Operation: Rehang or replace doors which do not swing or operate freely, as directed by Architect of Record.

Finished Doors: Refinish or replace doors damaged during installation, as directed by Architect of Record.

Institute protective measures as recommended and accepted by door manufacturer to ensure that wood doors are without damage or deterioration at time of final acceptance.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 08 1400, REV. 6, DATED APRIL 15, 2026.