Section 09 6500 
Resilient Flooring

  - ************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 6 Summary of Changes: Edited and updated as required for import into SpecLink.​

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects. For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 specifications and quality sections.

  - ************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain paragraphs for type of flooring based on project-specific requirements. Typically base at flooring finishes is required when sheet vinyl flooring, vinyl composition tile flooring and/or rubber tile flooring are in project-scope.

      - ***************************************************************************************

Sheet vinyl flooring.

Vinyl composition tile flooring.

Rubber tile flooring.

Stair coverings.

Base at flooring finishes.

RELATED REQUIREMENTS

Section << 01 8113.13 - Sustainable Design>>.

REFERENCE STANDARDS

ASTM D2047 - Standard Test Method for Static Coefficient of Friction of Polish-Coated Flooring Surfaces as Measured by the James Machine.

ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

ASTM E648 - Standard Test Method for Critical Radiant Flux of Floor-Covering Systems Using a Radiant Heat Energy Source.

ASTM F1066 - Standard Specification for Vinyl Composition Floor Tile.

ASTM F1303 - Standard Specification for Sheet Vinyl Floor Covering with Backing.

ASTM F1344 - Standard Specification for Rubber Floor Tile.

ASTM F1861 - Standard Specification for Resilient Wall Base.

ASTM F2169 - Standard Specification for Resilient Stair Treads.

EPA CPG - Comprehensive Procurement Guidelines for Construction Products.

SCAQMD 1168 - Adhesive and Sealant Applications.

      - ***************************************************************************************

      - The following Article Action Submittals is activated automatically based on product selections made in Article Materials and Accessories in PART 2, when applicable (for sustainable design requirements).

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

ACTION SUBMITTALS

      - ***************************************************************************************

      - Retain the following two paragraphs to request submittals if specific product &/or pattern & color specified in Article Materials are not used.

      - ***************************************************************************************

Product data

Samples of each pattern style and color range << specified; or  offered by the manufacturer>>.

      - ***************************************************************************************

      - Verify these requirements below against LANL ESM Chapter 14 - Sustainable Design, Attachment 1 "Required Attribute" Column and, if applicable, the LEED prerequisites and project credits. Material content and BioPreferred/recycled content required. The remaining submittals are designer-choices based on project-specific requirements. This paragraph with subparagraphs are activated automatically based on selections made in Article Materials and Accessories in PART 2, when applicable.

      - ***************************************************************************************

Sustainable Design Submittals:

Sustainable Design (SD) Submittals: Provide material content documentation in accordance with the requirements of this Section. ​​​​​​<< Submit any additional documentation for this product category, as applicable under Section  01 8113.13 - Sustainable Design , with this Section’s submittal.; or  None - N/A>>​​​​​​

Submit manufacturer’s documentation of BioPreferred/recycled content for each product.

Indoor Air Quality Certificates: Certify volatile organic compound content for each interior ​​<< adhesive; and  sealant>>​​​ and related primer.

INFORMATIONAL SUBMITTALS

Manufacturer’s installation instructions.

At Closeout:

Manufacturer’s cleaning and maintenance information.

      - ***************************************************************************************

      - Retain the following Article Extra Materials when required for project.

      - ***************************************************************************************

EXTRA MATERIALS

Provide at least << 5 percent; or  _______>> of calculated area of flooring materials for each type specified.

DELIVERY, STORAGE, AND HANDLING

Prior to installation, store materials for three (3) days in area of installation to achieve temperature stability. Allow seven (7) days for storage in winter months. Store materials in manufacturer’s shipping packages.

Maintain ambient temperature required by adhesive manufacturer 3 days prior to, during, and 24 hours after installation of adhesive materials.

Store highly flammable materials (adhesives, fillers, solvents) segregated from other materials.

PART 2 - PRODUCTS

MANUFACTURERS AND PRODUCTS

      - ***************************************************************************************

      - The following paragraphs are designer-choices. Retain paragraphs for type of flooring selected for project. Retain manufacturer(s) for each type of flooring required for project.

      - ***************************************************************************************

Sheet Vinyl with Backing

Armstrong World Industries, Inc.

Product: << _______>>​

Pattern Style: << ______>>​

Color: << _______>>​

Congoleum Corp.

Product: << _______>>

Pattern Style: << _______>>

Color: << _______>>​.

Mannington

Product: << _______>>​

Pattern Style: << _______>>​

Color: << _______>>

Vinyl Composition Tile

Armstrong World Industries, Inc.

Product: << _______>>​

Pattern Style: << _______>>

Color: << _______>>​

Congoleum Corp.

Product: << _______>>

Pattern Style: ​<< _______>>​

Color: << _______>>​

Mannington

Product: << ____>>​​

Pattern Style: << _______>>​

Color: << _______>>​

Rubber Tile

Johnsonite

Product:  << ____>>​

Pattern Style:  << _______>>​

Color:  << _______>>

Mannington

Product:  << ____>>​

Pattern Style:  << ____>>​

Color:  ​<< _______>>​​

Roppe

Product:  << _______>>

Pattern Style:   << _______>>​

Color:  << _______>>​

Stair Coverings

Burke Flooring Products

Product:  << _______>>​​

Pattern Style:  << _______>>​​

Color:  << _______>>​

Johnsonite

Product:  << _______>>​

Pattern Style:  << _______>>.

Color:  << _______>>​

R.C. Musson Rubber Co.

Product:  << _______>>​

Pattern Style:  << _______>>​

Color:  << _______>>​

Base Materials

Burke Flooring Products

Product:  << _______>>.

Pattern Style:  << _______>>​​

Color:   << _______>>​

Flexco Company

Product:  << _______>>​

Pattern Style:  << _______>>​

Color:  << _______>>​​

Roppe

Product:  << _______>>​​

Pattern Style:  << _______>>​

Color:  << _______>>​

MATERIALS

      - ***************************************************************************************

      - The following paragraphs are designer-choices. Retain paragraphs for type of flooring selected for project. Typically base at flooring finishes is required when sheet vinyl flooring, vinyl composition tile flooring and/or rubber tile flooring are in project-scope.

      - ***************************************************************************************

Sheet Vinyl with Backing:

<<ASTM F1303>>, Type II.

Flame spread/smoke developed rating of 25/50 or less in accordance with <<ASTM E84>>.

Vinyl wear thickness of 0.050 inch, total thickness of 0.080 inch; sheet width of 6 feet.

Vinyl Composition Tile:

<<ASTM F1066>>​, 1/8 inch thick x 12 inches x 12 inches.

Flame spread of not more than 75 in accordance with <<ASTM E84>>.

Critical radiant flux of not less than 0.45 W/sq cm in accordance with <<ASTM E648>>.

Sustainable Design Requirements: BioPreferred, if available or meet <<EPA CPG>> (90-100 percent recovered plastic).

Rubber Tile:

<<ASTM F1344>>; ​<< Class I - Homogenous; or  Class II - Heterogeneous (Layered)>>​, 1/8 inch thick x 12 inches x 12 inches.

Flame spread of not more than 75 in accordance with <<ASTM E84>>.

Critical radiant flux of not less than 0.45 W/sq cm according to <<ASTM E648>>.

Sustainable Design Requirements: BioPreferred, if available or meet <<EPA CPG>> (90-100 percent postconsumer rubber).

Stair Coverings:

Cured rubber compound meeting <<ASTM F2169>>.

Flame spread rating/smoke developed rating of 25/50 or less in accordance with <<ASTM E84>>.

Non-slip in accordance with <<ASTM D2047>>.

Sanded back; standard factory homogenous solid color throughout; and raised circular design.

Stair Treads: 1/4 inch thick at nose tapering back to rear edge, full width and depth of stair tread in one piece; return down edge of tread 1 1/2 inches to form nosing.

Stair Risers: Sheet material to match stair treads, 1/8 inch thick.

Stair Landing: Tile or sheet material to match stair treads, 1/8 inch thick.

Base: Material to match stair treads, 1/8 inch thick.

Base:

<<ASTM F1861>>, Type TS rubber.

Roll stock for distances 8 feet or greater, 4 inches high; 1/8 inch thick; ​​<< straight;  coved; or  butt-to>>​​ style.

ACCESSORIES

Sub-floor Filler: << Cementitious;  Premix latex; or  _______>>; type recommended by adhesive manufacturer.

Primers and Adhesives: Waterproof; types recommended by flooring manufacturer.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain when required for project.

        - ******************************************************************************

Sustainable Design Requirements for Adhesives: Maximum volatile organic compound content in accordance with <<SCAQMD 1168>>

Sealer and Wax: Types recommended by flooring manufacturer.

Moldings and Edge Strips: << Same material as flooring;  Metal;  _______; or  Enter Product Name/Part Number  manufactured by  Enter name of manufacturer>>.

Feature Strips: Same material as flooring, << _______>> inch wide.

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Retain the first paragraph when using sheet vinyl. Retain the second paragraph when using coved base.

      - ***************************************************************************************

Sheet Vinyl Flooring Welding Rod: Solid vinyl bead produced by vinyl flooring manufacturer for heat welding seams, in color ​​<< matching; or  contrasting with>>​​ field color.

Filler for Coved Base: ​​​<< Plastic;  Wood; or  ______>>​​​.

PART 3 - EXECUTION

EXAMINATION

Verification of Conditions

Verify that surfaces are smooth and flat with maximum variation of 1/8 inch in 10 feet and are ready to receive work.

        - ******************************************************************************

        - Retain the following subparagraph when concrete floor is present on project. Selecting concrete floor here updates rest of Section.

        - ******************************************************************************

Verify that concrete floors are dry with a 3 percent maximum moisture content, and exhibits negative alkalinity, carbonization, or dusting.

Verify that floor surfaces are free of substances that may impair adhesion of adhesive and finish materials.

Evaluation And Assessment

If existing conditions will adversely affect acceptable results, do not begin installation and notify LANL Subcontract Technical Representative (STR).

SURFACE PREPARATION

Thoroughly clean all surfaces to receive covering.

        - ******************************************************************************

        - Retain the following subparagraph when replacement of existing floor tile is in project-scope.

        - ******************************************************************************

Where replacement of existing floor tile is specified on the drawings, remove all traces of floor tile adhesive.

The floor surface temperature shall be 60 degrees F or higher.

      - ***************************************************************************************

      - The following two paragraphs are designer-choices. Retain the first paragraph when concrete floor is present on project. Retain the second paragraph when wood floor is present on project.

      - ***************************************************************************************

Concrete surfaces should be at least 30 days old and cleaned of all traces of curing compound. Smooth all roughness and uneven joints by fillers and/or grinding.

Sand wood floors smooth and make all joints even by ​<< fillers; or  sanding>>​.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain when required for project.

        - ******************************************************************************

Where specified on the drawings, install an underlayment of A-C exterior fir plywood, thickness as specified. Install plywood finish side up. Secure with barbed or serrated nails, using one fastener for every two (2) square feet.

INSTALLATION

Install in accordance with manufacturer’s installation instructions.

        - ******************************************************************************

        - Retain the following subparagraph when using sheet vinyl.

        - ******************************************************************************

For sheet vinyl, install with a minimum number of seams.

Spread only enough adhesive to permit installation of materials before initial set.

Set flooring in place and press with heavy roller to attain full adhesion.

Terminate flooring at centerline of door where adjacent floor finish is dissimilar.

Unless specifically shown otherwise on the Drawings, install tile with pattern running in same direction - no checker boarding.

Scribe flooring to walls, columns, cabinets, floor outlets and other appurtenances to provide tight joints.

      - ***************************************************************************************

      - Retain the following paragraph when using base materials.

      - ***************************************************************************************

Base:

Fit joints tight and vertical. Maintain minimum measurement of 18 inches between joints.

Miter internal corners. At external corners, "V" cut back of base strip to 2/3 of its thickness and fold.

Install base on solid backing. Bond tight to wall and floor surfaces.

Scribe and fit to door frames and other interruptions.

CLEANING AND PROTECTION

Keep surfaces of resilient flooring free of adhesives during installation. Remove excess adhesive from surface within recommended working time.

Remove soil, stain, and extraneous material caused by installation of resilient material from adjacent surfaces.

Clean and finish resilient flooring surfaces as recommended by the manufacturer. Remove and replace defective, off color, or improperly installed materials that cannot be made to satisfactorily match adjacent surfaces.

Protect the installed resilient material from damage. Do not permit traffic or work over the installed material until the material is adequately protected.

Prohibit traffic from areas with resilient flooring material for 24 hours after installation.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 09 6500, REV. 6, DATED APRIL 15, 2026.