Section 09 6816 
Sheet Carpeting

  - *************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 4 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - *************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

Sheet Carpeting.

Accessories.

RELATED REQUIREMENTS

Section << 01 2500 - Substitution Procedures>>

Section << 01 8113.13 - Sustainable Design>>

Section << 09 6813 - Tile Carpeting>>

    - **********************************************************************************************

      - The codes and standards listed below are provided for reference only and are not listed within the body of this Section. It is a designer-choice to include these codes/standards.

      - <<16 CFR 1630>>

      - <<AATCC Test Method 20>>

      - <<AATCC Test Method 165>>

      - <<AATCC Test Method 189>>

      - <<ASTM D3676>>

      - <<ASTM D3936>>

      - <<ASTM D5116>>

      - <<ASTM D5252>>

      - <<ASTM D5417>>

      - <<ASTM E662>>

      - <<CRI Test Method 101>>

      - <<IES TM-30>>

      - <<NFPA 101>>

    - **********************************************************************************************

REFERENCE STANDARDS

16 CFR 1630 - Standard for the Surface Flammability of Carpets and Rugs (FF 1-70).

AATCC Test Method 20 - Test Method for Fiber Analysis: Qualitative.

AATCC Test Method 134 - Test Method for Electrostatic Propensity of Carpets.

AATCC Test Method 165 - Test Method for Colorfastness to Crocking: Textile Floor Coverings - Crockmeter.

AATCC Test Method 189 - Test Method for Fluorine Content of Carpet Fibers.

AATCC TM16.3 - Test Method for Colorfastness to Light:  Xenon-Arc.

ABA Standards - ABA Accessibility Standards.

ASTM D1335 - Standard Test Method for Tuft Bind of Pile Yarn Floor Coverings.

ASTM D2859 - Standard Test Method for Ignition Characteristics of Finished Textile Floor Covering Materials.

ASTM D3676 - Standard Specification for Rubber Cellular Cushion Used for Carpet or Rug Underlay.

ASTM D3936 - Standard Test Method For Resistance To Delamination Of The Secondary Backing Of Pile Yarn Floor Covering.

ASTM D4157 - Standard Test Method for Abrasion Resistance of Textile Fabrics (Oscillatory Cylinder Method).

ASTM D4158 - Standard Guide for Abrasion Resistance of Textile Fabrics (Uniform Abrasion).

ASTM D5116 - Standard Guide for Small-Scale Environmental Chamber Determinations of Organic Emissions from Indoor Materials/Products.

ASTM D5252 - Standard Practice for the Operation of the Hexapod Tumble Drum Tester.

ASTM D5417 - Standard Practice for Operation of the Vettermann Drum Tester.

ASTM E648 - Standard Test Method for Critical Radiant Flux of Floor-Covering Systems Using a Radiant Heat Energy Source.

ASTM E662 - Standard Test Method for Specific Optical Density of Smoke Generated by Solid Materials.

CRI (GL) - Green Label Testing Program - Certified Products.

CRI (GLP) - Green Label Plus Testing Program - Certified Products.

CRI 104 - Standard for Installation of Commercial Carpet.

CRI Test Method 101 - Assessment of Carpet Surface Appearance Change using the CRI Reference Scales.

IES TM-30 - Technical Memorandum: IES Method for Evaluating Light Source Color Rendition.

ISO 2551 - Textile Floor Coverings and Textile Floor Coverings in Tile Form - Determination of Dimensional Changes Due to the Effects of Varied Water and Heat Conditions and Distortion Out of Plane.

NFPA 99 - Health Care Facilities Code.

NFPA 101 - Life Safety Code.

NFPA 253 - Standard Method of Test for Critical Radiant Flux of Floor Covering Systems Using a Radiant Heat Energy Source.

TTPIA - 15 USC 70, The Textile Products Identification Act.

      - ***************************************************************************************

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

SUBMITTALS

Action Submittals

Catalog data on carpet, describing physical<< and performance; or  None - N/A>> characteristics; sizes, patterns,<< colors available,; or  None - N/A>> and method of installation.

Manufacturer's certificate indicating that products meet or exceed the specified requirements and tests per Article << 2.3; or  _______>> Performance Requirements.

        - ******************************************************************************

        - Retain the following subparagraph when project requires samples.

        - ******************************************************************************

Samples: Four 12 by 12 inch samples illustrating proposed color and pattern for each carpet specified.

Shop drawings including seaming plan, method of joining seams, direction of carpet, ​​<< boundaries of different colors or patterns; and  _______>>​​.

        - ******************************************************************************

        - Verify these requirements below against LANL ESM Chapter 14 - Sustainable Design, Attachment 1 "Required Attribute" Column and, if applicable, the LEED prerequisites and project credits. Material content and BioPreferred/recycled content required. The remaining submittals are designer-choices based on project-specific requirements. This subparagraph with subparagraphs below it are activated automatically based on selections made in Article Performance Requirements in PART 2, when applicable.

        - ******************************************************************************

Sustainable Design Submittals:

Sustainable Design (SD) Submittals: Provide material content documentation in accordance with the requirements of this Section. ​​​​​​​​​​​​​​​​<< Submit any additional documentation for this product category, as applicable under Section  01 8113.13 - Sustainable Design , with this Section’s submittal.; or  None - N/A>>​​​​​​​​​​​​​​​​

Submit manufacturer’s documentation of BioPreferred/recycled content for each product.

          - **********************************************************************

          - The following subparagraph is a designer-choice. Retain when required for project.

          - **********************************************************************

Submit manufacturer’s documentation that carpet, cushion, adhesives and seam sealer meet the Sustainable Design requirements (Green Label Plus and Green Label certificates and VOC limits) per Article ​<< 2.3; or  _______>>​ Performance Requirements.

Informational Submittals:

Manufacturer’s installation instructions indicating procedures for installing carpet products and accessories.

At Closeout:

Maintenance Instructions: Submit manufacturer’s printed instructions for maintenance of installed work, including methods and frequency recommended for maintaining optimum condition under anticipated traffic and use conditions. Include precautions against materials and methods which may be detrimental to finishes and performance.

Warranty.

      - ***************************************************************************************

      - Retain the following Article Extra Materials when required for project.

      - ***************************************************************************************

EXTRA MATERIALS

Provide overrun of 5 percent of carpeting required (other than scrap pieces) for each color and type specified, for proper installation and unavoidable waste. Wrap overrun carpet for protection.

QUALITY ASSURANCE

Manufacturer Qualifications: Firm with not less than 5 years of production experience, whose published literature clearly indicates general compliance of products with requirements of this Section.

Installer Qualifications: Firm with not less than 5 years of experience in installation of commercial carpet of type, quantity and installation methods similar to work of this Section. Installer shall be approved by manufacturer.

Labeling: A label meeting the Federal Labeling Requirements, as stated in Federal Trade Commission's <<TTPIA>>, shall be attached to the certification samples and the products delivered.

All products within the same floor of a project shall be from the same dye lot.

DELIVERY, STORAGE, AND HANDLING

Comply with <<CRI 104>>, Section 4.0, “Storage and Handling”.

Prior to installation, store materials for three (3) days in area of installation to achieve temperature stability.

Maintain minimum 70 degrees F ambient temperature three days prior to, during, and 24 hours after installation of materials. In addition, maintain the relative humidity at approximate level at which the area is to be maintained.

WARRANTY

Provide written warranty for all labor and materials, signed by Subcontractor, Installer, and Manufacturer agreeing to repair or replace defective materials and workmanship of carpet installation during a ​<< 2; or  _______>>​ year period following final acceptance.

Provide non-prorated 15 year warranty against wear and lifetime warranty against loss of anti-static properties, from date of substantial completion.

PART 2 - PRODUCTS

    - **********************************************************************************************

      - The following Article Product Options and Substitutions is a designer-choice Retain when project allows for substitutions.

    - **********************************************************************************************

PRODUCT OPTIONS AND SUBSTITUTIONS

Alternate products may be accepted; follow Section << 01 2500 - Substitution Procedures>>.

MANUFACTURERS

Mannington Mills Inc.

Milliken Contract Carpets

Mohawk Group

Shaw Contract Group

<< _______>>

PERFORMANCE REQUIREMENTS

Compatibility: Provide carpet, adhesives, and other related materials that are compatible with one another and with substrates under conditions of service and application.

Physical Performance Requirements:

Carpet, including backing, shall pass methenamine pill test in accordance with <<ASTM D2859>>.

Carpet, including backing and accessories, shall have a minimum Class II rating, with a minimum critical radiant flux of 0.22 W/sq cm when tested according to <<ASTM E648>> (<<NFPA 253>>) Radiant Panel Test.

Carpet shall have an electrostatic propensity rating of no more than 3.0 kV when tested in accordance with <<AATCC Test Method 134>>.

        - ******************************************************************************

        - Retain the following subparagraph when Health Care Facilities are in project-scope. This is a requirements for those type of facilities.

        - ******************************************************************************

Carpet shall have an electrical resistance in the range of 10 to the ninth to 25,000 ohms based on <<NFPA 99>>.

Carpet shall pass Decay Test per <<AATCC Test Method 134>> for dissipating static electrical charges.

Carpet shall pass physical strength test per <<ASTM D1335>> with a tuft bind of 7 pounds.

Carpet shall complete abrasion test per <<ASTM D4157>> or <<ASTM D4158>> with no appreciable sign of pilling or fuzzing.

Carpet shall pass <<ISO 2551>> for dimensional stability.

Carpet shall have a grey scale rating of at least 4 after completing <<AATCC TM16.3>>.

      - ***************************************************************************************

      - The following two paragraphs are designer-choices. Retain based on project-specific conditions. If broadloom (sheet) carpet is to match carpet tile specified in Section << 09 6813 - Tile Carpeting>>, select applicable paragraph. Below is an example of one type of carpet to match Section << 09 6813 - Tile Carpeting>> and another to be specified in this Section.

        - A. Type A carpet shall be broadloom made of materials << and colors; or  None - N/A>>  to match carpet tile specified in Section << 09 6813 - Tile Carpeting>>​.

        - B. Type B carpet shall conform to the following criteria: [list criteria].

      - ***************************************************************************************

Material and Construction Requirements:<< Type  ____; or  None - N/A>> Carpet shall be broadloom made of materials<< and colors; or  None - N/A>> to match carpet tile specified in Section << 09 6813 - Tile Carpeting>>.

Material and Construction Requirements:<< Type  _______; or  None - N/A>> Carpet shall conform to the following criteria:

Pile Fiber: << BSF; or  ANTRON>> Type 6 or Type 6.6 nylon fiber.

Primary and Secondary Backing: Synthetic.

​Construction: Tufted or Fusion Bonded.

Yarn: Multi-ply.

Tufting Machine Gauge: Minimum ​<< 1/10; or  ______>>​ inch.

Number of Stitches: Minimum ​<< 8; or  ______>>​ per inch.

Pile Height: << 0.125 to 0.3; or  ___>> inch.

Face/Pile Weight: Minimum << 30; or  ______>> oz/sq yd.

Dye Method: Yarn or solution dyed.

Color and Pattern: Selected by the Architect and approved by LANL from Manufacturer's color line of product meeting this Section.

Sustainable Design Requirements:

BioPreferred/Recycled Content:

Nylon Carpet Face Fiber: BioPreferred. Minimum 7 percent biobased content.

Nylon Carpet Backing: For vinyl primary backing, use the highest percentage of total recovered materials content available; however, 0 percent is not acceptable.

Synthetic Fibers Carpet Cushion: 100 percent recovered carpet fabrication scrap.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain when required for project.

        - ******************************************************************************

Indoor Air Quality:

Carpet: Shall be only those certified with <<CRI (GLP)>>.

Cushion: Shall be only those certified with <<CRI (GL)>>.

Adhesive: VOC level shall not exceed 50 g/L.

Seam Sealer: VOC level shall not exceed 50 g/L.

Accessibility Requirements:

Carpet pile, transitions and floor slopes must comply with <<ABA Standards>>​.

ACCESSORIES

Floor Patching/Leveling Compound: << White premix latex; or  _______>>; type recommended by carpet manufacturer.

Adhesive: Permanent type adhesive as recommended by carpet manufacturer and that complies with <<CRI (GL)>> and flammability requirements for installed carpet.

Products

Durabond D-860.

Roberts 6300.

Henry 351 FastPro.

      - ***************************************************************************************

      - Retain the following paragraph when concrete floor is present on project.

      - ***************************************************************************************

Concrete Sealer: Provide concrete sealer compatible with carpet backing material and adhesive.

      - ***************************************************************************************

      - The following two paragraphs are designer-choices for edge guard type. Metallic edge guards are used in high-traffic or commercial areas where durability, impact resistance, and longevity are priorities - for example, at door thresholds, elevator lobbies, or transition zones between carpet and hard surfaces. Non-metallic edge guards are often used in office interiors, or areas of lower foot traffic where flexibility, cost efficiency, and aesthetics blending with flooring are more important.

      - ***************************************************************************************

Carpet Edge Guard, Metallic: Extruded aluminum bend-down type edge guard with concealed gripper teeth and minimum 1-1/2-inch-wide anchorage flange and minimum 5/8-inch-wide face flange. Provide with anodized aluminum finish of colors selected by the University from among standard colors available within the industry, (any manufacturer).

Carpet Edge Guard, Non-metallic: Extruded or molded heavy-duty vinyl or rubber carpet edge guard of size and profile indicated, with a minimum 2-inch-wide anchorage flange; colors selected by the University from among standard colors available within the industry (any manufacturer).

Vinyl Base: 4 inches high, coved, 0.080 inch gauge.

Miscellaneous Materials: As recommended by manufacturer of carpet and other carpeting products for intended application.

PART 3 - EXECUTION

    - **********************************************************************************************

      - Retain the following Article Carpet Removal if project involves removal of existing carpet.

    - **********************************************************************************************

CARPET REMOVAL

All carpet and scraps removed from LANL projects shall be recycled or reapplied.  Disposal of product in landfill is not permitted.

Vacuum in-place carpet with industrial vacuum prior to demolition or debris-producing renovation.

Remove carpet and form into rolls.

Stack carpet rolls for handling.

Store carpet off ground and under cover to prevent contamination of materials by water, foreign matter or other elements.

Coordinate with manufacturer for pickup for recycle. In the case that the manufacturer does not have a recycling program, contact an independent recycling program.

EXAMINATION

Examine substrates for moisture content, alkaline presence and other conditions on which carpet is to be installed.

Provide Calcium Chloride test results to verify amount of moisture in new or existing slabs.

Calcium Chloride shall be less than or equal to three (3) pounds per one thousand (1000) square feet in 24 hours.

Alkalinity testing shall result in a pH range of 5-9.

Notify LANL in writing of conditions detrimental to proper completion of the work.

Do not proceed until unsatisfactory conditions have been corrected.

Field-verify all dimensions and other work conditions affecting the installation of carpet.

Verify that substrate surfaces are smooth and flat with maximum variation of 1/8 inch in 10 feet and are ready to receive work before beginning installation.

PREPARATION

General: Comply with <<CRI 104>>, Section 7.0, “Site Conditions" and Section 8.0, "Substrate Preparation", and carpet manufacturer’s written installation instructions for preparing substrates indicated to receive carpet installation.

        - ******************************************************************************

        - Retain the following subparagraph when concrete floor is present on project.

        - ******************************************************************************

Concrete shall be cured, clean and dry before installation proceeds.

Repair minor holes, cracks, depressions, or rough areas using floor patching/leveling compound that is compatible with the adhesive to be used. Protrusions over 1/32 inch shall be leveled. Large, patched areas shall be primed.

Whenever a powdery or porous surface is encountered, a primer compatible with the adhesive shall be used to provide a suitable surface for glue-down installation.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain based on project-specific conditions.

        - ******************************************************************************

Remove existing utility caps and install grout fill caps; grout level with existing substrate.

Remove grease, oils, loose particles, paint, dirt, curing and parting agents, and all foreign substances. Clear away debris and scrape up cementitious deposits from surfaces to receive carpet; vacuum clean immediately before installation.

Subfloor shall be level and smooth and free from scaling and irregularities per <<CRI 104>>.

Coordinate carpet installation with any underfloor electrical/telephone/data for conductors and cables to be pulled prior to carpet installation.

Accessories shall be installed on top of carpet after installation.

Sequence carpet installation with other work to minimize possibility of damage and soiling of carpet during remainder of construction period.

      - ***************************************************************************************

      - The following four paragraphs are designer-choice. Retain the first or second paragraph when concrete floor is present on project based on project-specific conditions. Retain the third paragraph when wood floor is present on project. Retain the fourth paragraph when vinyl tile is present on project and requires replacement.

      - ***************************************************************************************

Concrete (No Previous Flooring Material): Apply a concrete sealer coat at the rate recommended by the sealer manufacturer. Use thin, fast drying primers that are compatible with adhesives. Apply adhesive only after primer is completely dry.

Concrete (Previous Flooring Material): Remove existing adhesive by grinding with a concrete grinding machine and moist sand. Do not use solvents to remove adhesive. Apply concrete sealer at rate recommended by sealer manufacturer. Use thin, fast drying primers that are compatible with adhesives. Completely cover area with cutback neutralizer per manufacturer’s recommendation.

Wood: Patch all surface irregularities and seal with wood sealer as recommended by manufacturer.

Vinyl Tile: Replace all loose, missing, and broken tiles with tiles of equal thickness making note of any special requirements with respect to asbestos. Strip existing tile of wax, dirt and other foreign substances. Completely cover area with cutback neutralizer per manufacturer’s recommendations.

Proceed with installation only after unsatisfactory conditions have been corrected.

INSTALLATION

General:  Comply with <<CRI 104>>, “Broadloom Carpet", and carpet manufacturer’s written instructions, except where more stringent requirements are shown or specified, and except where project conditions require extra precautions or provisions to ensure satisfactory performance of the Work.

Apply adhesive and seam sealer in accordance with manufacturer's installation instructions.

Extend carpet under open-bottom obstructions and under removable flanges and furnishings, and into alcoves and closets of each space.

          - **********************************************************************

          - The following subparagraph is a designer-choice. Retain based on project-specific conditions.

          - **********************************************************************

Existing systems furniture shall be raised or removed to accommodate continuous carpet installation.

Provide cut-outs where required, and bind cut edges where not concealed by protective edge guards or overlapping flanges.

        - ******************************************************************************

        - Retain the following subparagraph when carpet edge guard required for project.

        - ******************************************************************************

Install carpet edge guard where edge of carpet is exposed; anchor guards to substrate. Use full-length strips only.

Expansion Joints: Do not bridge building expansion joints with carpet; provide for movement.

Carpet shall be free from movement when subjected to traffic.

Verify carpet match to ensure minimal variation between dye lots before cutting.

Locate seams in area of least traffic.

Fit seams straight, not crowded or peaked, free of gaps.

Lay carpet on floors with run of pile in same direction as anticipated traffic.

Do not change direction of pile in any room where carpet is continuous through a wall opening into another room. Locate change of color or pattern between rooms under door centerline.

At floor finish material changes at doors, place centerline of abutting materials below door.

      - ***************************************************************************************

      - Retain one of the following two paragraphs based on installation method selected for project.

      - ***************************************************************************************

Glue-down Installation: Install per manufacturer's recommendations and <<CRI 104>>.

      - *** OR ***

Double glue-down Installation: Install per manufacturer's recommendations and <<CRI 104>>.

Install vinyl base after completion of carpet work. Use adhesive recommended by vinyl base manufacturer.

CLEANING AND PROTECTION

Remove and dispose of debris and recycle all unusable scraps.

Remove excess adhesive from floor, base, and wall surfaces without damage.

Clean and vacuum carpet surfaces.

Protect installed carpet to comply with <<CRI 104>>, Section ​​​<< 13.7; or  14.10>>​​​​, “Post Installation”.

Prohibit traffic from carpet areas for 24 hours after installation.

Protect installed carpet with a heavy, non-staining building paper. Do not use a moisture barrier such as plastic film. Maintain protection intact until final acceptance by LANL.

Advise LANL of protection methods and materials necessary to prevent deterioration or damage until substantial completion.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 09 6816, REV. 4, DATED APRIL 15, 2026.