Section 10 2113.13 
Metal Toilet Compartments

  - ************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 5 Summary of Changes: Edited and updated as required for import into SpecLink.​

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - ************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

Metal toilet compartments configured as:

        - ******************************************************************************

        - The following subparagraphs are designer-choice. Retain subparagraphs based on project-scope.

        - ******************************************************************************

Toilet enclosures

Entrance screens

Urinal screens

RELATED REQUIREMENTS

Section << 01 2500 - Substitution Procedures>>​

Section << 01 8113.13 - Sustainable Design>>

Section << 05 5000 - Metal Fabrications>>

Section << 06 1000 - Rough Carpentry>>

Section << 10 2800 - Toilet and Bath Accessories>>

REFERENCE STANDARDS

ABA Standards - ABA Accessibility Standards.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

ASTM A666/A666M - Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar.

ASTM A743/A743M - Standard Specification for Castings, Iron-Chromium, Iron-Chromium-Nickel, Corrosion Resistant, for General Application.

ASTM A879/A879M - Standard Specification for Steel Sheet, Zinc Coated by the Electrolytic Process for Applications Requiring Designation of the Coating Mass on Each Surface.

ASTM B26/B26M - Standard Specification for Aluminum-Alloy Sand Castings.

ASTM B86 - Standard Specification for Zinc and Zinc-Aluminum (ZA) Alloy Foundry and Die Castings.

ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

ASTM B455/B455M - Standard Specification for Copper-Zinc-Lead Alloy (Leaded-Brass) Extruded Shapes.

ASTM B584 - Standard Specification for Copper Alloy Sand Castings for General Applications.

ASTM F446 - Standard Consumer Safety Specification for Grab Bars and Accessories Installed in the Bathing Area.

EPA CPG - Comprehensive Procurement Guidelines for Construction Products.

ICC A117.1 - Accessible and Usable Buildings and Facilities.

      - ***************************************************************************************

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

ACTION SUBMITTALS

      - ***************************************************************************************

      - Verify these requirements below against LANL ESM Chapter 14 - Sustainable Design, Attachment 1 "Required Attribute" Column and, if applicable, the LEED prerequisites and project credits. Material content is required. This paragraph is activated automatically based on selections made in Article Materials in PART 2, when applicable.

      - ***************************************************************************************

Sustainable Design (SD) Submittals: Provide material content documentation in accordance with the requirements of this Section. ​​​​<< Submit any additional documentation for this product category, as applicable under Section  01 8113.13 - Sustainable Design , with this Section’s submittal.; or  None - N/A>>​​​​

Product Data: For each type of product.

Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for toilet compartments.

Shop Drawings: For toilet compartments:

Include plans, elevations, sections, and attachment details.

Show locations of cutouts for compartment-mounted toilet accessories.

Show locations of reinforcements for compartment-mounted grab bars and locations of blocking for surface-mounted toilet accessories.

Show locations of centerlines of toilet fixtures.

Show locations of floor drains.

Show​​<< ceiling grid, ceiling-mounted items, and; or  None - N/A>>​​​ overhead support or bracing locations.

      - ***************************************************************************************

      - Retain the following two paragraphs when project requires samples.

      - ***************************************************************************************

Samples for Initial Selection: For each type of toilet compartment material indicated.

Include Samples of hardware and accessories involving material and color selection.

Samples for Verification: For the following products, in manufacturer's standard sizes unless otherwise indicated:

Each type of material, color, and finish required for toilet compartments, prepared on 6-inch square Samples of same thickness and material indicated for Work.

Each type of hardware and accessory.

Product Schedule: For toilet compartments, prepared by or under the supervision of supplier, detailing location and selected colors for toilet compartment material.

INFORMATIONAL SUBMITTALS

Product Certificates: For each type of toilet compartment.

​At Closeout:

Maintenance Data: For toilet compartments to include in maintenance manuals.

      - ***************************************************************************************

      - The following Article is a designer-choice. Retain when extra materials are required for project.

      - ***************************************************************************************

EXTRA MATERIALS

Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Door Hinges: One hinge with associated fasteners.

Latch and Keeper: One latch and keeper with associated fasteners.

        - ******************************************************************************

        - Retain the following subparagraph when required for project. Selecting subparagraph "Door Bumper" here updates the rest of the Section.

        - ******************************************************************************

Door Bumper: One door bumper with associated fasteners.

Door Pull: One door pull with associated fasteners.

Fasteners: 10 fasteners of each size and type.

DELIVERY, STORAGE, AND HANDLING

Deliver and store material in the manufacturer’s shipping packages.

Protect material from damage.

Handle material carefully to prevent damage. Unpack and install in a manner to protect the panel finish from damage.

FIELD CONDITIONS

Field verify actual locations of toilet fixtures, walls, columns, ceilings, and other construction contiguous with toilet compartments before fabrication.

PART 2 - PRODUCTS

MANUFACTURERS

All American Metal Corporation

Global Steel Products Corporation, Embassy

General Partitions Mfg. Corporation

Sanymetal Products Company, Inc.

PRODUCT OPTIONS AND SUBSTITUTIONS

Alternate products may be accepted; follow Section << 01 2500 - Substitution Procedures>>.

REGULATORY REQUIREMENTS

Comply with applicable provisions in <<ABA Standards>> and <<ICC A117.1>> for toilet compartments designated as accessible.

MATERIALS

      - ***************************************************************************************

      - Select one of the following paragraphs based on material selected for toilet compartments for project.

      - ***************************************************************************************

Aluminum

Aluminum Castings: <<ASTM B26/B26M>>.

Aluminum Extrusions: <<ASTM B221>>.

      - *** OR ***

Brass

Brass Castings: <<ASTM B584>>.

Brass Extrusions: <<ASTM B455/B455M>>.

      - *** OR ***

Steel Sheet: Commercial steel sheet for exposed applications; mill phosphatized and selected for smoothness.

        - ******************************************************************************

        - Select one of the following two coatings/finish based on project-specific requirements.

        - ******************************************************************************

Electrolytically Zinc Coated: <<ASTM A879/A879M>>, 01Z.

        - *** OR ***

Hot-Dip Galvanized: <<ASTM A653/A653M>>, either hot-dip galvanized or galvannealed.

Sustainable Design Requirements: Refer to <<EPA CPG>>​.

      - *** OR ***

Stainless Steel

Stainless-Steel Sheet: <<ASTM A666/A666M>>, Type 304, stretcher-leveled standard of flatness.

Stainless-Steel Castings: <<ASTM A743/A743M>>.

Sustainable Design Requirements: Refer to <<EPA CPG>>.

      - *** OR ***

Zamac: <<ASTM B86>>, commercial zinc-alloy die castings, chrome plated.

METAL TOILET COMPARTMENTS

      - ***************************************************************************************

      - The following three paragraphs are designer-choice. Retain paragraphs based on project-scope.

      - ***************************************************************************************

Toilet Enclosure Style: ​​​​​<< Overhead braced;  Floor anchored;  Ceiling hung; or  Floor and ceiling anchored>>​​​​​.

Entrance Screen Style: ​​​<< Overhead braced;  Floor anchored;  Ceiling hung; or  Floor and ceiling anchored>>​​​.

Urinal Screen Style: ​​​​<< Wall hung, flat panel;  Wall hung with integral flanges;  Wall hung, wedge shaped;  Floor anchored;  Overhead braced; or  Post to ceiling>>​​​​.

Door, Panel, and Pilaster Construction: Seamless, metal facing sheets pressure laminated to core material; with continuous, interlocking molding strip or lapped-and-formed edge closures; corners secured by welding or clips and exposed welds ground smooth.<<  Provide with no-sightline system.; or  None - N/A>> Exposed surfaces shall be free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.

Core Material: Manufacturer's standard sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch for doors and panels and 1-1/4 inches for pilasters.

        - ******************************************************************************

        - Retain the following subparagraph for accessible toilet compartments requiring grab bars.

        - ******************************************************************************

Grab-Bar Reinforcement: Provide concealed internal reinforcement for grab bars mounted on units of size and material adequate for panel to withstand applied downward load on grab bar of at least 250 lbf (1112 N), when tested according to <<ASTM F446>>​, without deformation of panel.

Tapping Reinforcement: Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to units.

Facing Sheets and Closures: Hot-dip galvanized-steel sheet with nominal base-metal (uncoated) thicknesses as follows:

Pilasters, Braced at Both Ends: Manufacturer's standard thickness, but not less than 0.036 inch.

Pilasters, Unbraced at One End: Manufacturer's standard thickness, but not less than 0.048 inch.

Panels: Manufacturer's standard thickness, but not less than 0.030 inch.

Doors: Manufacturer's standard thickness, but not less than 0.030 inch.

      - ***************************************************************************************

      - Selecting the applicable option in the paragraph below updates the rest of the Section.

      - ***************************************************************************************

Pilaster ​​​​<< Shoes;  Sleeves (Caps); or  Shoes and Sleeves (Caps)>>​​​​: Stainless-steel sheet, not less than 0.031-inch nominal thickness and 3 inches high, finished to match hardware.

Posts: Manufacturer's standard post design of material matching the thickness and construction of pilasters, square with ​<< shoe;  sleeve (cap); or  shoe and sleeve (cap)>>​  matching that on the pilaster.

Brackets (Fittings):

        - ******************************************************************************

        - Select one of the following bracket types based on project-specific requirements.

        - ******************************************************************************

Stirrup Type: Ear or U-brackets; ​​​​​<< chrome-plated zamac;  clear-anodized aluminum;  stainless steel; or  chrome-plated brass>>​​​​​.

        - *** OR ***

Full-Height (Continuous) Type: Manufacturer's standard design; ​​​<< stainless steel; or  aluminum>>​​​.

      - ***************************************************************************************

      - Retain one of the following two paragraphs based on project-specific requirements. Retain the first paragraph for wood supports. Retain the second paragraph for metal supports. Retain the subparagraphs within it that apply based on project-specific requirements.

      - ***************************************************************************************

Refer to Section ​​​<< 06 1000 - Rough Carpentry>>​​​ for:

Blocking.

Overhead support of floor and ceiling anchored compartments.

Overhead support of post to ceiling screens.

      - *** OR ***

Refer to Section << 05 5000 - Metal Fabrications>> for supports that attach the following to the overhead structural system:

Ceiling hung compartments.

Floor and ceiling anchored compartments.

Post to ceiling screens.

HARDWARE AND ACCESSORIES

      - ***************************************************************************************

      - Retain one of the following two paragraphs based on project-specific requirements. Retain the first paragraph for standard operating hardware and accessories. Retain the second paragraph for heavy-duty hardware and accessories.

      - ***************************************************************************************

Hardware and Accessories: Manufacturer's standard operating hardware and accessories.

Material: ​​​​<< Chrome-plated zamac;  Clear-anodized aluminum;  Stainless steel; or  Chrome-plated brass>>​​​​.

Hinges: Manufacturer's standard << paired, self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees;  continuous, cam type that swings to a closed or partially open position; or  continuous, spring-loaded type>>, allowing emergency access by lifting door.

Latch and Keeper: Manufacturer's standard << recessed; or  surface-mounted>> latch unit designed for emergency access and with combination rubber-faced door strike and keeper. Provide units that comply with regulatory requirements for accessibility at compartments designated as accessible.

        - ******************************************************************************

        - The following two subparagraphs are designer-choices. Retain the first subparagraph when coat hook required for project. Retain the second subparagraph when door bumper required for project.

        - ******************************************************************************

Coat Hook: Manufacturer's standard combination hook and rubber-tipped bumper, sized to prevent in-swinging door from hitting compartment-mounted accessories.

Door Bumper: Manufacturer's standard rubber-tipped bumper at out-swinging doors ​<< and entrance-screen doors; or  None - N/A>>​.

Door Pull: Manufacturer's standard unit at out-swinging doors that complies with regulatory requirements for accessibility. Provide units on both sides of doors at compartments designated as accessible.

      - *** OR ***

Hardware and Accessories: Manufacturer's heavy-duty operating hardware and accessories.

Hinges: Manufacturer's minimum 0.062-inch thick, stainless-steel ​<< paired, self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees;  continuous, cam type that swings to a closed or partially open position; or  continuous, spring-loaded type>>​, allowing emergency access by lifting door. Mount with through-bolts.

Latch and Keeper: Manufacturer's heavy-duty, surface-mounted, cast stainless-steel latch unit designed to resist damage due to slamming, with combination rubber-faced door strike and keeper and with provision for emergency access. Provide units that comply with regulatory requirements for accessibility at compartments designated as accessible. Mount with through-bolts.

        - ******************************************************************************

        - The following two subparagraphs are designer-choices. Retain the first subparagraph when coat hook required for project. Retain the second subparagraph when door bumper required for project.

        - ******************************************************************************

Coat Hook: Manufacturer's heavy-duty, combination cast stainless-steel hook and rubber-tipped bumper, sized to prevent in-swinging door from hitting compartment-mounted accessories. Mount with through-bolts.

Door Bumper: Manufacturer's heavy-duty, rubber-tipped, cast stainless-steel bumper at out-swinging doors<< and entrance-screen doors; or  None - N/A>>. Mount with through-bolts.

Door Pull: Manufacturer's heavy-duty cast stainless-steel pull at out-swinging doors that complies with regulatory requirements for accessibility. Provide units on both sides of doors at compartments designated as accessible. Mount with through-bolts.

Refer to Section << 10 2800 - Toilet and Bath Accessories>> for toilet tissue dispensers, grab bars, purse shelves, and similar accessories mounted on toilet compartments.

      - ***************************************************************************************

      - Retain the following paragraph when overhead bracing is required depending on the toilet compartment type selected.

      - ***************************************************************************************

Overhead Bracing: Manufacturer's standard continuous, metal head rail with anti-grip profile and in manufacturer's standard finish.

Anchorages and Fasteners: Manufacturer's standard exposed fasteners of stainless steel, finished to match the items they are securing, with theft-resistant-type heads. Provide sex-type bolts for through-bolt applications. For concealed anchors, use stainless steel, hot-dip galvanized steel, or other rust-resistant, protective-coated steel compatible with related materials.

FABRICATION

Fabrication, General: Fabricate toilet compartment components to sizes indicated. Coordinate requirements and provide cutouts for through-partition toilet accessories, and solid blocking within panel where required for attachment of toilet accessories.

      - ***************************************************************************************

      - The following six paragraphs are designer-choice. Retain paragraphs based on the toilet compartment type selected for project.

      - ***************************************************************************************

Overhead Braced Units: Provide manufacturer's standard corrosion-resistant supports, leveling mechanism, and anchors at pilasters to suit floor conditions. Provide ​<< shoes;  sleeves (caps); or  shoes and sleeves (caps)>>​ at pilasters to conceal supports and leveling mechanism.

Floor Anchored Units: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment nuts at pilasters for structural connection to floor. Provide << shoes;  sleeves (caps); or  shoes and sleeves (caps)>>  at pilasters to conceal anchorage.

Wall Hung Units: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment nuts at pilasters for connection to structural support at walls. Provide << shoes;  sleeves (caps); or  shoes and sleeves (caps)>> at tops of pilasters to conceal anchorage.

Ceiling Hung Units: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment nuts at pilasters for connection to structural support above finished ceiling. Provide assemblies that support pilasters from structure without transmitting load to finished ceiling. Provide ​<< shoes;  sleeves (caps); or  shoes and sleeves (caps)>>​ at tops of pilasters to conceal anchorage.

Floor and Ceiling Anchored Units: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment at tops and bottoms of pilasters. Provide << shoes;  sleeves (caps); or  shoes and sleeves (caps)>> at pilasters to conceal anchorage.

Post to Ceiling Anchored Units: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment at tops and bottoms of pilasters. Provide << shoes;  sleeves (caps); or  shoes and sleeves (caps)>> at pilasters to conceal anchorage.

Posts: Provide manufacturer's standard corrosion-resistant anchoring assemblies with leveling adjustment at ​​<< tops and; or  None - N/A>>​​ bottoms of posts. Provide ​​<< shoes;  sleeves (caps); or  shoes and sleeves (caps)>>​​ at posts to conceal anchorage.

Door Size and Swings: Unless otherwise indicated, provide 24-inch wide, in-swinging doors for standard toilet compartments and 36-inch wide, out-swinging doors with a minimum 32-inch wide, clear opening for compartments designated as accessible.

PART 3 - EXECUTION

EXAMINATION

Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for fastening, support, alignment, operating clearances, and other conditions affecting performance of the Work.

Confirm location and adequacy of blocking and supports required for installation.

Proceed with installation only after unsatisfactory conditions have been corrected.

Coordinate layout and installation of supports, inserts, and anchors built into other units of work for toilet compartment anchorage.

INSTALLATION

General: Comply with manufacturer's written installation instructions. Install units rigid, straight, level, and plumb. Secure units in position indicated with manufacturer's recommended anchoring devices.

Maximum Clearances

Pilasters and Panels: 1/2 inch.

Panels and Walls: 1 inch.

        - ******************************************************************************

        - Select one of the following subparagraphs for installation based on bracket type selected for project.

        - ******************************************************************************

Stirrup Brackets: Secure panels to walls and to pilasters with no fewer than ​​​<< two brackets attached; or  three brackets attached at midpoint and>>​​​ near top and bottom of panel.

          - **********************************************************************

          - The following subparagraph is a designer-choice. Retain based on project-specific conditions.

          - **********************************************************************

Locate wall brackets such that holes for wall anchors occur in concrete or CMU or masonry or tile joints.

Align brackets at pilasters with brackets at walls.

        - *** OR ***

Full-Height (Continuous) Brackets: Secure panels to walls and to pilasters with full-height brackets.

          - **********************************************************************

          - The following subparagraph is a designer-choice. Retain  based on project-specific conditions.

          - **********************************************************************

Locate bracket fasteners such that holes for wall anchors occur in concrete or CMU or masonry or tile joints.

Align brackets at pilasters with brackets at walls.

      - ***************************************************************************************

      - Retain one of the following two paragraphs based on project-specific requirements. Retain the first paragraph for installation of wood supports. Retain the second paragraph for installation of metal supports. Retain the subparagraphs within it that apply.

      - ***************************************************************************************

Refer to Section ​​​<< 06 1000 - Rough Carpentry>>​​​ for installation of:

Blocking.

Overhead support of floor and ceiling anchored compartments.

Overhead support of post to ceiling screens.

      - *** OR ***

Refer to Section ​<< 05 5000 - Metal Fabrications>>​ for installation of supports that attach the following to the overhead structural system:

Ceiling hung compartments.

Floor and ceiling anchored compartments.

Post to ceiling screens.

      - ***************************************************************************************

      - The following seven paragraphs are designer-choice. Retain paragraphs for installation based on the toilet compartment types selected for project.

      - ***************************************************************************************

Overhead Braced Units: Secure pilasters to floor and level, plumb, and tighten. Set pilasters with anchors penetrating not less than 1-3/4 inches (44 mm) into structural floor unless otherwise indicated in manufacturer's written instructions. Secure continuous head rail to each pilaster with no fewer than two fasteners. Hang doors to align tops of doors with tops of panels and adjust such that tops of doors are parallel with overhead brace when doors are in closed position.

Floor Anchored Units: Set pilasters with anchors penetrating not less than 2 inches (51 mm) into structural floor unless otherwise indicated in manufacturer's written instructions. Level, plumb, and tighten pilasters. Hang doors and adjust such that tops of doors are level with tops of pilasters when doors are in closed position.

Wall Hung Units: Secure pilasters to supporting structure and level, plumb, and tighten. Hang doors and adjust such that bottoms of doors are level with bottoms of pilasters when doors are in closed position.

Ceiling Hung Units: Secure pilasters to supporting structure and level, plumb, and tighten. Hang doors and adjust such that bottoms of doors are level with bottoms of pilasters when doors are in closed position.

Floor and Ceiling Anchored Units: Secure pilasters to supporting construction and level, plumb, and tighten. Hang doors and adjust such that doors are level and aligned with panels when doors are in closed position.

Post to Ceiling Anchored Units: Secure pilasters to supporting construction and level, plumb, and tighten. Hang doors and adjust such that doors are level and aligned with panels when doors are in closed position.

Screens: Attach with anchoring devices to suit supporting structure. Set units level and plumb, rigid, and secured to resist lateral impact.

ADJUSTING

Hardware Adjustment: Adjust and lubricate hardware according to hardware manufacturer's written instructions for proper operation. Set hinges on in-swinging doors to hold doors open approximately 30 degrees from closed position when unlatched. Set hinges on out-swinging doors << and doors in entrance screens; or  None - N/A>>  to return doors to fully closed position.

CLEANING

Remove protective masking and clean surfaces.

Replace damaged or scratched materials with new materials.

END OF SECTION

    - *****************************************************************************************

      - Do not omit the following reference information:

    - *****************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 10 2113.13, REV. 5, DATED APRIL 15, 2026.